Release CC100_Driver_3110_V2.7.8-4_3010_V2.7.5-2

Information

- Product Code:		CC100

- Firmware Version:	v2.7.8-4

File Name:		CC100_Driver_3110_V2.7.8-4_3010_V2.7.5-2

- Date:			12Sep2008

Upgrade Instructions

1.	With the CC100 connected to your PC, manually configure your LAN IP address to 192.168.1.x.

2.	Run any Web browser, like Microsoft Internet Explorer. Go to “http://192.168.1.1 to access the CC100 home page.

3.	Click “Upload Firmware” then “Upload” and wait for few seconds.

4.	Click “continue” to locate the latest Firmware and press start and press “continue” again to complete the upload.

5.	To upgrade all the Endpoint CC101, go to “EP management”.

6.	Tick the box for the CC100 as well as the any CC101 in the table for firmware upgrade and press “upgrade” on the bottom of the page.

7.	Press Upgrade again on the Upgrade Driver page to complete the upgrade process.

New Features

- Improved Robustness through Multiple Retransmissions

- To improve robustness and PER performance multiple retransmissions of packets are now supported. By default, a packet requiring retransmission will be sent several times (more than once).

Known Issues

- Minimum attenuation required between any two nodes is 8dB

- v2.7.8 backward compatibility –Packet sent to host more than once. Due to support for multiple retransmissions, when working with older versions of the CG3110M or older chip (CG3010M) the last packet in a stream may be sent to the host (by the receiver) more than once.

Release CC100_Driver_3110_V2.7.6-7_3010_V2.7.5-2

Information

- Product Code:		CC100

- Firmware Version:	v2.7.6-7

File Name:		CC100_Driver_3110_V2.7.6-7_3010_V2.7.5-2

- Date:			14May2008

Upgrade Instructions

1.	With the CC100 connected to your PC, manually configure your LAN IP address to 192.168.1.x.

2.	Run any Web browser, like Microsoft Internet Explorer. Go to “http://192.168.1.1 to access the CC100 home page.

3.	Click “Upload Firmware” then “Upload” and wait for few seconds.

4.	Click “continue” to locate the latest Firmware and press start and press “continue” again to complete the upload.

5.	To upgrade all the Endpoint CC101, go to “EP management”.

6.	Tick the box for the CC100 as well as the any CC101 in the table for firmware upgrade and press “upgrade” on the bottom of the page.

7.	Press Upgrade again on the Upgrade Driver page to complete the upgrade process.

New Features

- Downstream and Upstream Uni-cast PE is limited to be at least 16/2. This prevents the negotiated payload encoding (PE) or PHY rate from dropping to less than 32Mbps. This will protect the network from degraded performance in the presence of a device on a bad communication link. The packet errors will be corrected by the LARQ mechanism.

Fixes:

  - Sometimes, the master could not manage the Rate Negotiation to all EPs, leading to the wrong PE decision.

  - Sometimes, there is a mistake in the PER calculation that lead to the wrong PE decision.

  - When the EP’s HOST table overflowed, the EP did not pass Multicast data.

  - The link led does not blink on Tx packets, only on Rx packets.

  - Dynamic config - Master doesn't download configuration to all EPs. This bug is more visible when loading a large number of EPs (e.g. 32 EPs).

Release CC100_Driver_3110_V2.7.5-2_3010_V2.7.5-2

Information

- Product Code:		CC100

- Firmware Version:	v2.7.5-2

File Name:		CC100_Driver_3110_V2.7.5-2_3010_V2.7.5-2

- Date:			25Sep2007

Upgrade Instructions

1.	With the CC100 connected to your PC, manually configure your LAN IP address to 192.168.1.x.

2.	Run any Web browser, like Microsoft Internet Explorer. Go to “http://192.168.1.1 to access the CC100 home page.

3.	Click “Upload Firmware” then “Upload” and wait for few seconds.

4.	Click “continue” to locate the latest Firmware and press start and press “continue” again to complete the upload.

5.	To upgrade all the Endpoint CC101, go to “EP management”.

6.	Tick the box for the CC100 as well as the any CC101 in the table for firmware upgrade and press “upgrade” on the bottom of the page.

7.	Press Upgrade again on the Upgrade Driver page to complete the upgrade process.

New Features

- Coax MDU supports new chip CG3110.

Release CC100_Driver_3110_V2.7.5-0

Information

- Product Code:		CC100

- Firmware Version:	v2.7.5-2

File Name:		CC100_Driver_3110_V2.7.5-0

- Date:			11June2008

Upgrade Instructions

1.	With the CC100 connected to your PC, manually configure your LAN IP address to 192.168.1.x.

2.	Run any Web browser, like Microsoft Internet Explorer. Go to “http://192.168.1.1 to access the CC100 home page.

3.	Click “Upload Firmware” then “Upload” and wait for few seconds.

4.	Click “continue” to locate the latest Firmware and press start and press “continue” again to complete the upload.

5.	To upgrade all the Endpoint CC101, go to “EP management”.

6.	Tick the box for the CC100 as well as the any CC101 in the table for firmware upgrade and press “upgrade” on the bottom of the page.

7.	Press Upgrade again on the Upgrade Driver page to complete the upgrade process.

New Features

- Coax MDU supports up to 31 EPs connected to the Master.

Fixes:

- Minimum PE on multicast transmission was not enforced correctly.

Release CC100_Driver_3110_V2.7.4-0

Information

- Product Code:		CC100

- Firmware Version:	v2.7.5-2

File Name:		CC100_Driver_3110_V2.7.4-0

- Date:			27April2007

Upgrade Instructions

1.	With the CC100 connected to your PC, manually configure your LAN IP address to 192.168.1.x.

2.	Run any Web browser, like Microsoft Internet Explorer. Go to “http://192.168.1.1 to access the CC100 home page.

3.	Click “Upload Firmware” then “Upload” and wait for few seconds.

4.	Click “continue” to locate the latest Firmware and press start and press “continue” again to complete the upload.

5.	To upgrade all the Endpoint CC101, go to “EP management”.

6.	Tick the box for the CC100 as well as the any CC101 in the table for firmware upgrade and press “upgrade” on the bottom of the page.

7.	Press Upgrade again on the Upgrade Driver page to complete the upgrade process.

New Features

- The EP-EP security is done in the physical layer.

- The EP configuration can be dynamically managed via the master by the master or by external host. With this feature, you don't need to update EP's driver after modify configuration. This feature must works with CEM-336 system firmware V2.10. It will be available in near future.

- V2.7.4 supports identification and marking of customized traffic types based on configurable traffic classifier rules and support for priority-based queue management and media access. Priority support is provided for up to 8 priority levels.

- Support LARQ

Fixes:

- There are no limitations on EP to EP attenuation.

Release CC100_Driver_3110_V1.7.20

Information

- Product Code:		CC100

- Firmware Version:	v2.7.5-2

File Name:		CC100_Driver_3110_V1.7.2

- Date:			15June2005

Upgrade Instructions

1.	With the CC100 connected to your PC, manually configure your LAN IP address to 192.168.1.x.

2.	Run any Web browser, like Microsoft Internet Explorer. Go to “http://192.168.1.1 to access the CC100 home page.

3.	Click “Upload Firmware” then “Upload” and wait for few seconds.

4.	Click “continue” to locate the latest Firmware and press start and press “continue” again to complete the upload.

5.	To upgrade all the Endpoint CC101, go to “EP management”.

6.	Tick the box for the CC100 as well as the any CC101 in the table for firmware upgrade and press “upgrade” on the bottom of the page.

7.	Press Upgrade again on the Upgrade Driver page to complete the upgrade process.

New Features

- Support high band (12~28).

Fixes:

- Unexpected (self) PER in some multi stream scenarios.

- Improved robustness in noisy environments.

- Handling of VLAN-tagged control messages.