NetComm Limited

Event Script Programmers Guide

Firmware Version 2.20

Nov.20th.2002

Table of Contents

1.	Introduction	5

2.	Event Script Format	6

3.	Comments	7

4.	Trigger Condition	8

4.1.	Time and Date Event Fields	8

4.2.	Trig source and Run Option Field	8

5.	Action Commands	10

5.1.	alert	11

5.2.	buffer	12

5.2.1.	buffer_init	12

5.2.2.	buffer_del	12

5.2.3.	buffer_start	12

5.2.4.	buffer_stop	13

5.3.	ftp	16

5.4.	log	19

5.5.	mail	20

5.6.	ptz	22

5.7.	relay	23

5.8.	reset	24

5.9.	sleep	25

5.10.	led	26

5.11.	motion	27

5.11.1.	motion_init	27

5.11.2.	motion_del	27

5.11.3.	motion_start	27

5.11.4.	motion_stop	28

5.12.	serial	29

5.13.	http	30

5.14.	PPP (Beta Version)	30

6.	Related CGI Commands	33

6.1.	Get Script File	33

6.2.	Set Script File	33

6.3.	Get Script Command Return Code	33

6.4.	Set Script Boot Flag	33

6.5.	Start Script	34

6.6.	Stop Script	34

6.7.	Check Script Status	34

6.8.	Show Log File	34

7.	Server Push Image Request	35

7.1.	HTTP URL request format	35

7.2.	HTTP traffic dump example	35

8.	Examples:	37

Document History

| Version | Date | Comment |
| --- | --- | --- |
| 2.10 | 2002-July-25 | Initial version. Support firmware since version 2.10 |
| 2.14 | 2002-Aug-30 | - 1. FTP will make new directory when no such path/file exist, path/file name up to 255 chars, and file name up to 30 chars supported. - 2. Supports up to 6 concurrently SLEEP command. - 3. Add MOTION_STOP command to avoid frequently running of MOTION_INIT. Refer to motion_stop - 4. Correct CGI getimage function framedelay. Refer to HTTP URL request format |
| 2.20 | 2002-Nov-20 | Support PPP (Beta Version) since version 2.20 Refer to PPP (Beta Version) |

# 1.   Introduction

The Event Script provides a flexible way for Administrator to configure NetComm NetComms as automatic intelligent devices in applications and a comprehensive set of “trigger conditions” and “action commands” allowed to fit in varies of applications without limitation.

Note: The event script is supported since firmware version 2.0; all models being NS4000/4100/4120/4240.

# 2.   Event Script Format

A event script for the NetComm IP Camera includes one or several entries and is entered into the editor in the following format:

-          # <comments>

-          <triggering condition> :

-          <action commands>;

-          %

where:

-          The <comments> are optional.

-          The <trigger condition> includes the following fields: <minute> <hour> <day> <month> <weekday> <optional trigger source and run option>

-          The <commands> include one or several event script action commands.

# 3.   Comments

It is a good programming practice to start each new entry with a comment to describe its function. Comments are optional but must be proceeded by a # character, as detailed below:

# <comment>

Example:

# This programming entry will...

# 4.   Trigger Condition

The commands contained within each specific entry are triggered by a defined trigger condition. The trigger condition is specified by six separate fields and must be terminated with a colon ":".

<Minute> <Hour> <Day> <Month> <Day of the week> <(optional) Trig Source and Run Option Field >:

## 4.1. Time and Date Event Fields

The first five fields specify the time and date events, i.e. Minute, Hour, Day, Month and Day of the week.

The syntax for each field within a trigger condition is governed by the following rules:

-          Each time and date field can contain several numerical event variables that are delimited by commas and hyphens.

-          Each field is delimited by an open space.

-          An asterisk (*) represents the full range of event variables within the relative time and date field, i.e. “* * * * *” means every minute, every hour, every day, every month, every day of the week.

-          Numerical event variables separated by a hyphen indicate an inclusive range; for example, 2-6 means 2 to 6.

Example:

Trigger every month, between the fourth and eighth at 10.03, 12.03 and 14.03, using the 24-hour clock:

3 10,12,14 4-8 * * :

## 4.2. Trig source and Run Option Field

The sixth field is a trigger source and runs option field that defines the trigger source other than timer and event run options.

The table below outlines the available trigger variables:

| Trigger | Description |
| --- | --- |
| RESET | Run the action when system resets |
| RUN | Run the action when script starts running |
| \V1 | Video channel one sync signal lose |
| /V1 | Video channel one sync signal back |
| \V2 | Video channel two sync signals lose |
| /V2 | Video channel two sync signals back |
| \V3 | Video channel three sync signals lose |
| /V3 | Video channel three sync signals back |
| \V4 | Video channel four sync signals lose |
| /V4 | Video channel four sync signals back |
| \I1 | GPIN port one falling (high to low) edge |
| /I1 | GPIN port one rising (low to high) edge |
| HI1 | GPIN port one high level (*1) |
| LI1 | GPIN port one low level (*1) |
| \I2 | GPIN port two falling (high to low) edge |
| /I2 | GPIN port two rising (low to high) edge |
| HI2 | GPIN port two high level (*1) |
| LI2 | GPIN port two low level (*1) |
| \I3 | GPIN port three falling (high to low) edge |
| /I3 | GPIN port three rising (low to high) edge |
| HI3 | GPIN port three high level (*1) |
| LI3 | GPIN port three low level (*1) |
| \I4 | GPIN port four falling (high to low) edge |
| /I4 | GPIN port four rising (low to high) edge |
| HI4 | GPIN port four high level (*1) |
| LI4 | GPIN port four low level (*1) |
| \N | Network link lose connection |
| /N | Network link back connection |
| M1 | Motion Detection of video channel one |
| M2 | Motion Detection of video channel two |
| M3 | Motion Detection of video channel three |
| M4 | Motion Detection of video channel four |
| G0 | CGI to trigger, http://IP/cgitrig?cmd=0 |
| G1 | CGI to trigger, http://IP/cgitrig?cmd=1 |
| G2 | CGI to trigger, http://IP/cgitrig?cmd=2 |
| G3 | CGI to trigger, http://IP/cgitrig?cmd=3 |
| G4 | CGI to trigger, http://IP/cgitrig?cmd=4 |
| G5 | CGI to trigger, http://IP/cgitrig?cmd=5 |
| G6 | CGI to trigger, http://IP/cgitrig?cmd=6 |
| G7 | CGI to trigger, http://IP/cgitrig?cmd=7 |
| G8 | CGI to trigger, http://IP/cgitrig?cmd=8 |
| G9 | CGI to trigger, http://IP/cgitrig?cmd=9 |

Note *1: The GPIN level trigger could be cascaded with other trigger variable with ‘&’ symbol, e.g. HI1&/I2&LI3, when rising edge of I2 occurs, if both I1 is high and I3 is low level at that time, then proceed the action node.

# 5.   Action Commands

The table below outlines the available action commands

| ALERT | Alert message to remote site through socket call |
| --- | --- |
| BUFFER | Ring Buffer activities include _init, _start, _stop, _del |
| FTP | Ftp to remote site |
| LOG | Log message to internal file |
| MAIL | Email to remote site |
| PTZ | Pan Tilt Zoom device control through serial port |
| RELAY | Relay activities |
| RESET | Invoke system reset |
| SLEEP | Produce delay in specified event |
| LED | LED indicator activities |
| MOTION | Motion detection activities include _init, _start, _stop, _del |
| SERIAL | Serial port message send |
| HTTP | Server push video stream send only when triggered |

A comprehensive catalog of available programming commands, complete with their syntax and valid options are described below:

## 5.1. alert

The alert command sends a message to a remote host.

##### Syntax

alert [-host HOST] [-port PORTNUMBER] [-message STRING];

##### Options

-host 
Specify the host IP address of the remote host. 
-port
Specify the TCP port number in the range [0..65534]. 
-message
Specify the message that will be sent to the remote host. The message must be surrounded by quotation marks. Hexadecimal coded messages must be preceded by \0x, e.g. "\0x04" for the non-printed character EOT. To include a quotation mark, it must be preceded by a backslash, e.g. \".

Example

# This command will send the message "Alarm # "0013" from NetComm 4000" to the remote host when # input port 1 goes high. * * * * * /I1: alert -host 172.21.1.202 -port 2703 -message "Alarm \"0013\" from NetComm 4000"; %

## 5.2. buffer

### 5.2.1.     buffer_init

The buffer_init command initiates and allocates memory space for the image buffer. This is needed in order to store images using the buffer_start and buffer_stop commands.

##### Syntax

buffer_init [-cam VIDEOCHANNEL] [-fmt VIDEOFORMAT] 
[-imagenum IMAGENUMBER];

##### Options

-cam
Specify the camera source, i.e. 1- 4
-fmt
Specify the image format (HxV), where: 
 	qsif=176x112, 
 	sif=352x240 and 
 	full=704x480
-imagenum
Specify the maximum number of images that can be stored in the buffer. The valid range is 1-100, although in practice is limited by the amount of available memory within the NetComm IP Camera.

### 5.2.2.     buffer_del

The buffer_del command frees all allocated memory space of the image buffer. None of argument needed.

Example 1

# At startup, this command will initiate one # buffer for camera 1 with 25 sif images # and one buffer for camera 2 with 5 full # images. * * * * * RUN: buffer_init –cam 1 –fmt sif –imagenum 25; buffer_init –cam 2 –fmt full –imagenum 5; %

Example 2

# This command will clear the buffer every Sunday. 0 0 * * 0 * : buffer_del; %

### 5.2.3.     buffer_start

The buffer_start command stores snapshots in a specific image buffer. Before the command can be used, the buffer facility must be initiated using the buffer_init command.

When invoked, the buffer_start command is executed in the background and any subsequent command in the event entry is executed immediately.

Images are retrieved using the ftp or mail or server push CGI command.

##### Syntax

buffer_start [-cam VIDEOCHANNEL] [-fmt VIDEOFORMAT]
[-interval TIME] [-duration TIME];

##### Options

-cam
Specify the camera source, i.e. 1- 4
-fmt
Specify  the image format (HxV), where: 
 	qsif=176x112, 
 	sif=352x240 and 
 	full=704x480
-interval
Specify the time interval between consecutive snapshots in unit of 10ms system tick. If omitted, default value is 50 (i.e. 500ms).
-duration
Specify the total time for the command to run. Accept range is
-1: 	Infinite loop,
0: 	Run Once,
>0: 	Value in unit of 10ms system tick
If omitted, the default value is 0 which will be issued only once. If you specify –1 (infinite loop), the command will continue to be executed until it is interrupted by any of the commands: buffer_del, buffer_start or buffer_stop.

Example

# This command will keep loading full size images. # For every 30 seconds * * * * * * : buffer_start -fmt full -cam 3 -interval 3000 -duration -1; %

### 5.2.4.     buffer_stop

The buffer_stop command terminates the storage of snapshots to an image buffer. The buffer_stop command will not block the execution of any subsequent commands in the event entry until completely terminated. Note that the buffer_start command behaves differently.

The command is typically used together with the buffer_start command.

##### Syntax

buffer_stop [-cam VIDEOCHANNEL] [-fmt VIDEOFORMAT]
[-interval TIME] [-duration TIME] [-imagenum SIZE];

##### Options

-cam
Specify the camera source, i.e. 1- 4
-fmt
Specify the image format (HxV), where: 
 	qsif=176x112, 
 	sif=352x240 and 
 	full=704x480
-interval
Specify the time interval between consecutive snapshots in unit of 10ms system tick. If omitted, default value is 50 (i.e. 500ms).
-duration
Specify the total time for the command to run. Accept range is
>0: 	Value in unit of 10ms system tick
=0: 	Specified the value with (interval X imagenum).
If omitted, the default value is 0 which will calculate by (interval X imagenum), the command will continue to be executed until it is interrupted by any of the commands: buffer_del, buffer_start.
-imagenum
Specify the number of images to be stored in the buffer. This setting will be taken into account only when the duration is 0 or omit. The maximum number of images is specified by the buffer_init command.

Example

# This command initiates the buffer to hold 60 # qsif and 5 full images. New images # are captured until the buffer_start command is # interrupted. * * * * * RUN : buffer_init –cam 1 –fmt qsif –imagenum 60; buffer_init –cam 1 –fmt full –imagenum 5; buffer_start -fmt qsif –cam 1 -interval 100 -duration -1; buffer_start -fmt full –cam 1 -interval 6000 -duration -1; % # When port 1 is triggered, an alert message is sent # to host 1.2.3.4. Another 10 qsif images are # captured with interval 500ms and all the images are sent to an ftp # server with the names "qsif_1" to # "qsif_60". Then the buffer is restarted. * * * * * /I1: alert -host 1.2.3.4 -port 4000 -message "Alert! Input on port 1."; buffer_stop -fmt qsif –cam 1 -imagenum 10; ftp -host somehost -user USER -pass PASS -fmt qsif –cam 1 –filename qsif_$r1-60 -buffer; alert -host 1.2.3.4 -port 4000 -message "Alert! Images sent to ftp server"; % # When port 2 is triggered, the 5 most recent # full images are sent to an ftp server # with the names "Picture_1" to "Picture5". Note that # the buffer is not stopped. * * * * * /I2: ftp -host somehost -user USER -pass PASS -fmt full –cam 1 –filename Picture_$r1-5 -buffer; %

## 5.3. ftp

The ftp command uses the standard File Transfer Protocol (FTP) for transferring an image from your NetComm IP Camera to a remote host.

##### Note:

The ftp command keeps the connection open during each file transfers.

##### Syntax

ftp [-host HOST] [-user USERNAME] [-pass PASSWORD] [-filename DESTINATIONFILE][-cam VIDEOCHANNEL] 
[-fmt VIDEOFORMAT] [-interval TIME] [-duration TIME] [-imagenum IMAGENUMBER] [-buffer];

##### Options

-host 
Specify the IP address of the remote host (ftp server).
-user 
Specify the user name for logging on to the remote host.
-pass
Specify the password for logging on to the remote host. 
-filename
Specify the name and whole path of the destination file. The maximum length of a destination path/file name is 255 characters; simple file name without path must be in 30 characters. 
You can add time stamps, such as time, date and file indexes, to the destination path/file name. This is useful for example when you are transferring a series of files. The following table lists the available time stamp variables. Every substitution must be proceeded by a dollar sign "$".

$s
The current second in the range 00-59.
$m
The current minute in the range 00-59.
$h
The current hour in the range 00-23.
$d
The current day of the month in the range 01-31.
$n
The current month in the range 01-12.
$y
The current year without the century in the range 00-99.
$Y
The current year including the century.
$rS-E
Inserts an index number starting from number S through all the numbers including the number E. If you leave out number E, it will be interpreted as a very large number.

-cam
Specify the camera source, i.e. 1- 4
-fmt
Specifies the image format (HxV), where: 
 	qsif=176x112, 
 	sif=352x240 and 
 	full=704x480
-interval
Specify the time interval between consecutive snapshots in unit of 10ms system tick. If omitted, default value is 50 (i.e. 500ms).
-duration
Specify the total time for the command to run. Accept value is
-1: 	Infinite loop,
0: 	Run Once,
>0: 	Value in unit of 10ms system tick
If omitted, the default value is 0 which will be issued only once. If you specify –1 (infinite loop), the command will continue to executed until stop running of event script.
-imagenum
Specify the images numbers to FTP
0 or Omit: FTP the images with above duration setting.
>0: Continue FTP until reaching specified image number.
-buffer 
Specify the ftp operation retrieve files from Ring Buffer. If omit, will capture the newest image to FTP.

Example 1

# This command will log in as "user" with the # password "pass" on the host with the Internet # address 123.123.123.123, download a full # image with the name "user.jpg" and put # it in the directory "home/user" on the remote # host. * * * * * : ftp -host 123.123.123.123 -user user -pass pass -fmt full –cam 1 -filename home/user/user.jpg; %

Example 2

# This command will every hour, every five seconds # for twenty minutes, download a sif image # from camera 3 and save the images with a time # stamp in the filename. 0 * * * * : ftp -host 192.168.0.1 -user user -pass pass -fmt sif -filename tmp/sif$h$m$s.jpg -interval 500 -duration 120000 -cam 3; %

Example 3

# This command will send 1001 consecutive sif # images to "your.host.name". The first and last # images will be named "sifsize1000.jpg" and # "sifsize2000.jpg", respectively. * * * * * : ftp -host 192.168.0.1 -user user -pass pass -fmt sif –cam 1 -filename home/camera1/tmp/sifsize$r1000-2000.jpg; %

## 5.4. log

This command writes a message to the NetComm IP Camera log file.

##### Note:

To display the log file, you type the following in the address/location field of your Web browser: http://IP/evscript?showlog=filename , where filename indicate the LOGFILENAME below.

##### Syntax

log [-filename LOGFILENAME] 
[-message LOGMESSAGE];

##### Options

-filename
Specifies the file name to log, note that the file will put in “Sys” directory.
-message
The message string must be surrounded by quotation marks.

Example:

# This command writes a message to the log file when the NetComm IP CAMERA # Network is lose. * * * * * \N : log "The Network lose link." ; %

## 5.5. mail

This command uses the Simple Mail Transfer Protocol (SMTP) for sending images and files from your NetComm IP CAMERA to e-mail addresses.

##### Note:

The IP address of the mail server and a valid return address on the TCP/IP page must be specified when using the mail command.

##### Syntax

mail [-host SMTPSERVER] [-filename ATTACHFILENAME] [-msgbody MAILMESSAGEBODY] [-returnaddr RETURNADDR] [-recipient RECIPIENTS] [-subject MAILSUBJECT]
[-cam VIDEOCHANNEL] [-fmt VIDEOFORMAT] [-interval TIME] [-duration TIME] [-imgnum ATTACHNUMBEROFIMAGE] [-buffer];

##### Options

-host:
Specifies the SMTP Server name or IP address, e.g. “mail.NetComm.com” or “61.220.235.170”
-filename
Attachment filename, such as FTP filename, accept time stamp character variable.
-msgbody
Text message contain in email body.
-returnaddr
Return mail address, if omit, default is webcam@NetComm.com.
-recipient
Destination address to receive this email, could be multiple address separate by semicolon “;”.
-subject
Specifies email subject string
-cam
Specifies the camera source, i.e. 1- 4
-fmt
Specifies the image format (HxV), where: 
 	qsif=176x112, 
 	sif=352x240 and 
 	full=704x480
-interval
Specifies the time interval between consecutive snapshots in unit of 10ms system tick. If omitted, default value is 50 (i.e. 500ms).
-duration
Specifies the total time for the command to run. Accept value is
-1: 		Infinite loop,
Omit or 0: 	Run Once,
>0: 		Value in unit of 10ms system tick
If omitted, the default value is 0 which will be issued only once. If you specify –1 (infinite loop), the command will continue to executed until stop running of event script.
-imagenum
Specifies the image number of attachment to mail
0 or Omit: If buffer disable, just mail the message body without attachment, if buffer enable, mail the complete Ring Buffer as attachment.
>0: If buffer enable, Mail with imagenum of attachment, if buffer disable, this setting will force to one attachment and mail frequency determined by above interval/duration setting.
-buffer 
Specifies the mail operation retrieve files from Ring Buffer. If omit, will capture the newest image to mail. If enable buffer, the interval and duration will disable which just mail once with imagenum of attachment from Ring Buffer.

Example 1

# This command will send an e-mail with the # subject "Hello" and the attached file # sif size to someone@company.com * * * * * : mail -subject Hello -fmt sif –cam 1 -recipient someone@company.com -host mail.company.com –filename test.jpg –imagenum 1; %

Example 2

# This command will send an e-mail with the # subject "This "sif size" was sent from my # NetComm IP CAMERA" to two recipients. * * * * * : mail -subject "This \"fullsize.jpg\" was sent from my NetComm IP CAMERA" -fmt sif –cam 1 -recipient “someone@NetComm.com; anotherone@NetComm.com” –host mail.NetComm.com; %

Example 3

# This command will send an e-mail with two attachments from buffer. * * * * * : mail –fmt sif –cam 1 –host mail.home -recipient me@home -imagenum 2 -buffer; %

## 5.6. ptz

This command send PTZ control protocol through serial port. Supported model of PTZ devices should be configured within administration page in advance.

##### Syntax

ptz [-cam VIDEOCHANNEL] [-speed SPEED] [-preset PRESET] [-pan –1|+1] [-tilt –1|+1] [-zoom –1|+1];

##### Options

-cam
Specifies the camera source, i.e. 1- 4.
-speed
Sets the move speed of PTZ camera, range 1(slowest) ~ 9(fastest).
-preset
Move PTZ camera to preset position, range 1~20.
-pan
Specified the pan factor of PTZ camera, -1 means pan left, and +1 means pan right.
-tilt
Specified the tilt factor of PTZ camera, -1 means tilt down, and +1 means tilt up.
-zoom
Specified the zoom factor of PTZ camera, -1 means zoom in, and +1 means zoom out.

Note that above PTZ control (preset, pan, tilt, zoom) should be run just one option at a time, multiple options may produce unpredictable result.

Example

# This command sets the PTZ camera to preset position 5 when GPIO Input # one is triggered: * * * * * /I1 : ptz –cam 1 –preset 5; % #This command pan left with middle speed for every one minute * * * * *: ptz –cam 1 –speed 5 –pan –1; %

## 5.7. relay

This command sets the relay on or off within the NetComm IP CAMERA.

##### Syntax

relay [-on | -off | -toggle | -onoff ONTIME | -offon OFFTIME];

##### Options

-on
sets the relay on.
-off
sets the relay off.
-toggle
Toggle the relay
-onoff
Set the relay to on when start trigger, after ONTIME, set back to off, ONTIME is time in unit of 10ms system tick.
-offon
Set the relay to off when start trigger, after OFFTIME, set back to on, OFFTIME is time in unit of 10ms system tick.

Example

# This command sets the relay ON and after 3 seconds back to OFF # when digital input 1 goes high: * * * * * /I1 : relay –onoff 300; %

## 5.8. reset

This command resets the NetComm IP CAMERA

##### Syntax

reset [-flag FLAG];

##### Options

-flag
Specifies to reboot flag
0:		Do Nothing
1:		Reboot system immediately

Example

# This command will reset the hardware once a day. 0 0 * * * * : reset –flag 1; %

## 5.9. sleep

This command adds a pause to the programming script execution. The command is typically used if you want to assure that the previous command is finished before continuing the script.

##### Syntax

sleep [-time PAUSETIME];

##### Options

-time
Specifies the time in unit of 10ms system tick to wait before proceeding with the next action command in the event entry.

Example

# This command will result in a 30 seconds pause in the programming script execution. * * * * * * : sleep –time 3000; %

## 5.10.        led

This command change the status of LED indicator, there are two indicators in NetComm IP CAMERA, one is Power/Status, the other is Network, both indicator could be change by Action commands

##### Syntax

led [-on | -off | -toggle | -norm | -blinkfast | -blinkslow]

##### Options

-on:
Set the P/S LED to on state, the color became orange.
-off:
Set the P/S LED to off state, the color become red.
-toggle
Toggle the P/S LED between orange and red.
-norm
Set both the P/S and Network LED back to factory default state.
-blinkfast
Set the Network LED to fast blinking.
-blinkslow
Set the Network LED to slow blinking.

## 5.11.        motion

This command control the motion detection mechanism, in priors to generate motion detection trigger, must enable (init and start) the motion detection in background task. And due to the motion detection take system overhead and resources, when no use of motion detection, could be disable (del).

Note: Before start up event script, must select the detection area in user interface of motion detection setting, otherwise below motion script won’t take into action.

### 5.11.1.               motion_init

The motion_init command initiates and allocate memory space for the motion detection image buffer. This is needed in order to detect image change using the motion_start commands.

##### Syntax

motion_init [-cam VIDEOCHANNEL] [-sensitivity SENSITIVITY];

##### Options

-cam
Specifies the camera source, i.e. 1- 4
- sensitivity
Specifies the sensitivity of motion detection, omit will set to default value as 5, set less for more sensitive.

### 5.11.2.               motion_del

The motion_del command free all allocated memory space and stop running task of the motion detection. None of argument needed.

### 5.11.3.               motion_start

The motion_start command start the motion detection task in background. Once detect the difference between images, will activate action command following motion trigger.

##### Syntax

motion_start [-cam VIDEOCHANNEL] [-interval TIME] 
[-duration TIME] [-holdtime TIME];

##### Options

-cam
Specifies the camera source, i.e. 1- 4
-interval
Specifies the time interval between consecutive snapshots in unit of 10ms system tick. If omitted, default value is 50 (i.e. 500ms).
-duration
Specifies the total time for the command to run. Accept range is
Omit or –1 or 0: 	Infinite loop,
>0: 			Value in unit of 10ms system tick
If you specify infinite loop, the command will continue to be executed until it is interrupted by motion_start, motion_del.
-holdtime
Specifies the pause time when detect the image changes, after the holdtime, will restart the motion detection task, if omit, normally image change will detect twice (IN and OUT of object), this option used to improve system performance, and to avoid double alarm when detect the image changes.

Example

# This command will init motion detection when start event * * * * * RUN: motion_init -cam 1; motion_start –cam 1 –interval 500 –duration –1; %

### 5.11.4.               motion_stop

For real-time system, to avoid consuming system performance, or, to stop motion detection in certain period, use motion_stop to suspend the motion detection task.

It’s useful when event frequently suspended and reactivated of motion detection.

##### Syntax

motion_stop [-cam VIDEOCHANNEL];

##### Options

-cam
Specifies the camera source, i.e. 1- 4

Example

# This command will init motion detection when start event * * * * * RUN: motion_init -cam 1; motion_start –cam 1 –interval 500 –duration –1; % # Will suspend motion detection for 10 seconds when GPIN 1 triggered * * * * * /I1: motion_stop –cam 1; sleep –time 1000; motion_start –cam 1 –interval 500 –duration –1; %

## 5.12.        serial

The serial command sends a message to serial com port.

##### Syntax

serial [-comport COM] [-message STRING];

##### Options

-comport
Specifies the com port in range [1..2]. if omit, default is com2. 
-message
Specifies the message that will be sent to the remote host. The message must be surrounded by quotation marks. Hexadecimal coded messages must be preceded by \0x, e.g. "\0x04" for the non-printed character EOT. To include a quotation mark, it must be preceded by a backslash, e.g. \".

Example

# This command will send the message "Alarm # "0013" from NetComm 4000" to com2 when # input port 1 goes high. * * * * * /I1: serial -port 2 -message "Alarm \"0013\" from NetComm 4000"; %

## 5.13.        http

The http command build an identifier used for server push image request, and indicate the connection information. Please refer to chapter Server Push Image Request, for more regarding server push. This command could send out images thought server push image request (natively supported by Netscape) only when trigger comes.

##### Syntax

http [-id IDENTIFIER] [-delay TIME] [-interval TIME][-duration TIME] [-buffer];

Where
-id
Specifies HTTP image request identifier, only identifier match and trigger comes, then feed out video stream thought HTTP. Valid range from 0~9. e.g. http://IP/getimage?id=0
-delay
Specifies delay time between frames when -buffer is turn on, where
=0:	No delay (default)
>0:	Value in unit of 10ms system tick.
-interval
Only valid without -buffer option, specifies the time interval between consecutive snapshots in unit of 10ms system tick. Default value is 50 (i.e. 500ms).
-duration
Only valid without -buffer option, specifies the total time for the command to run. Accept range is
Omit or 0: 	Just once,
–1:		Infinite loop,
>0: 		Value in unit of 10ms system tick

Example

#Activate when motion detected * * * * * M1: #Send out complete buffer image thought HTTP id 0, delay 100ms between images #This could be pre Alarm images http –id 0 –buffer –delay 10; #Send out currently images for 10 seconds, interval 250ms (4 fps) #This could be post Alarm images http –id 1 –interval 25 –duration 1000; %

## 5.14.        PPP (Beta Version)

The ppp command can dial up to your ISP (Internet Service Provider) so that a serial-line TCP/IP connection can be established. This dial up is made through external modem, so COM1 of NetComm IP CAMERA should be connected to external modem, which is dialed through a regular telephone line with an ISP account.

##### Syntax

ppp [-phone PHONE_NUMBER] [-user USERNAME] [-pass PASSWORD] [-close];

Where
-phone
Specifiy ISP phone number which you’d like to dial up. 
-user
Specifiy the username so that you can log on your ISP dial-up account.
-pass
Specifiy the password so that you can log on your ISP dial-up account.
-close
To disconnect the PPP and hang up the phone.

##### Note:

- 1. In this beta version, ppp connections and disconnections are limited no more than 3 times. If exceeding this number of times, please restart the system and try again. - 2. Before issue PPP -close, you must use “sleep” command to reserve enough time for action nodes to complete transmission.

Examples

# PPP Demostration1 # Dial up to connect an ISP to establish a PPP connection if GPIN4 is from High to Low. # Then send a mail with a picture attached. # The system will pause for 3 seconds to send one mail and then disconnect. * * * * * \I4 : ppp -phone 4125678 -user username -pass password; mail -host mail.NetComm.com -msgbody hello -recipient someone@NetComm.com -returnaddr receiver@NetComm.com -subject "I4 triggered" -cam 1 -fmt sif -imagenum 1 -filename I4$h$m$s.jpg; sleep -time 3000; ppp -close; % # PPP Demostration2 # Dial up to connect an ISP to establish a PPP connection if ethernet lose link. # Then send a mail with a picture attached. # Reset system when ethernet link is back. * * * * * \N : ppp -phone 4125678 -user username -pass password; mail -host mail.NetComm.com -msgbody hello -recipient someone@NetComm.com -returnaddr receiver@NetComm.com -subject "Network Lose" -cam 1 -fmt sif -imagenum 1 -filename N$h$m$s.jpg; % * * * * * /N: reset –flag 1; %

# 6.   Related CGI Commands

To access NetComm IP CAMERA event system from CGI command which provide convenient way to integrate from System Integrator.

The CGI syntax like http://IP/evscript?[cmd], for example, if NetComm IP CAMERA IP address is 192.168.0.101, then enter http://192.168.0.101/evscript?runcmd=check will get result code regarding event parsing / running / bootflag status.

The event script represented as file contain in NetComm IP CAMERA “Sys/scr00.txt” user could either update the file by FTP upload, or submit from CGI command http://IP/evscript?setscript=EVENTFILE , below descript all event script related CGI command.

Note: To save any changes of the script and flag into flash mush click “save changes” in user interface, or enter http://IP/eepromsave CGI, otherwise will restore the old one after reset.

## 6.1. Get Script File

##### Syntax

#### http://IP/evscript?getscript

This command load the script file “Sys/scr00.txt” and response to browser as simple text

## 6.2. Set Script File

##### Syntax

#### http://IP/evscript?setscript=EVENTFILE

This command receive the EVENTFILE and save as “Sys/scr00.txt” , then do the parsing job, below command will query the parsing result.

## 6.3. Get Script Command Return Code

##### Syntax

#### http://IP/evscript?getresult

This command return the result of last parsing or start event result, to get correct result, recommend delay one second before issue this CGI command to allow NetComm IP CAMERA complete the parsing or event startup job, the return will be:
OK
Fail
or Text which indicate the begging of syntax error in script file.

## 6.4. Set Script Boot Flag

##### Syntax

#### http://IP/evscript?setboot=[0/1]

This command set the boot flag to indicate NetComm IP CAMERA to startup event after reset.
0:	Do not start up event
1: Start up event after reset

## 6.5. Start Script

##### Syntax

#### http://IP/evscript?runcmd=start

This command will load the script file, parsing, and start running the event script.

## 6.6. Stop Script

##### Syntax

#### http://IP/evscript?runcmd=stop

This command will stop the running script.

## 6.7. Check Script Status

##### Syntax

#### http://IP/evscript?runcmd=check

This command will return the status of event system. As below:
Parsing=0/1:		1 means parsing done, 0 means not yet parsing or err
RunStatus=0/1:	1 means event started, 0 means event stop
BootFlag=0/1:	1 means start event after reset, 0 means not.

## 6.8. Show Log File

##### Syntax

#### http://IP/evscript?showlog=FILENAME

This command will return the log file specified by FILENAME

# 7.   Server Push Image Request

The server push method of image request will establish an image link with NetComm IP CAMERA, and NetComm IP CAMERA will active continue send out streaming MJPEG images and don’t need client site to send further request until connection break.

Facilitate the server push CGI image request to combine with event trigger, such as to send only “buffer image” as pre-alarm, and image when “motion detected” as post-alarm. This mode used for real time recording to reduce tremendous network traffic and disk space. Also due to HTTP server push performance is much higher than FTP and SMTP, this mode could be more efficient for system integrator to use for real time storage.

## 7.1. HTTP URL request format

##### Request Syntax

http://IP/getimage?camera=[SRC]&fmt=[SIZE]&delay=[TIME]&id=[0~9]

Where
camera	:	Specifies the camera source, i.e. 1- 4, omit default is 1.
fmt:		Specifies the image format (HxV), omit default is sif, which: 
 	qsif=	176x112, 
 	sif=	352x240 and 
 	full=	704x480
delay: Specifies the time delay between frames, omit default is 0, which 
 	0 or Omit= No delay
 	>0=		Frame delay value in unit of 10ms system tick
 	Note that if there are HTTP ID action nodes (refer to http ) correspond to this CGI
 	id (see below) with delay option, then the “delay” in HTTP id action node will 
 	override this “delay” setting when HTTP id trigger comes.
id:		Specifies the http action id in event script, omit default send 
 			continue MJPEG images, Which
 	0~9=		Http action identifier, if event is started with HTTP action node, then
 			the image stream will pause until trigger comes and matching id inside 
 			the action node.
 	else=	If event script stopped, send out continue images.

## 7.2. HTTP traffic dump example

- a)       Client send server-push image request to Server
GET /getimage?camera=1&fmt=sif HTTP/1.0

- b)       Server send reply 
HTTP/1.0 200 Okay
Date: Thu, 01 Jan 1970 00:00:00 GMT
Connection: close
Content-Type: multipart/x-mixed-replace;boundary=--myboundary

- c)       Server send streaming video image header
--myboundary
Content-Type: image/jpeg
Content-Length: 008867 Stamp:07d20401 0011360e 09 0000084b

Where 
--myboundary: frame boundary between images
Content-Type: identify of jpeg
Content-Length: size of following jpeg image
Stamp: Time stamp, where format with “Date”, “Time”, “Tick”, and “Sequence-number”
 		
Date:	Field 			Bits		Example
 		Year, A.D. 		31-16	07d2=2002 AD
 		Month (1-12) 		15-8		04=Apr
 		Day (1-31) 		7-0		01=First
 		
Time:	Field 			Bits
 		Hour (0-23) 		31-16	0011=17 hr
 		Minute (0-59) 		15-8		36=54 min
 		Second (0-59) 		7-0		0e=14 sec
 		
Tick:	Field 			Bits
 		Ticks(0-99)		7-0 		09=90 ms from last sec.
Seq:	Field 			Bits
 		Seq (0-4294927696)	31-0 	84b=2123 images since server start

- d)       Server sending JPEG images which start from 0xffd8, and end with 0xffd9

- e)     Goto step c) until connection interrupted by client side or no video signal detected.

# 8.   Examples:

Below example demonstrate miscellaneous event including ring buffer and motion detection, then request using server-push method with Netscape.

#Event1 #every minutes LED toggle * * * * *: LED -toggle; % #Event2 #GPIN4 falling to toggle realy->wait 3 second -> toggle relay * * * * * G0: relay -toggle; sleep -time 300; relay -toggle; % #Event3 #GPIN3 falling to toggle relay->FTP with interval 1 second, duration 10 seconds * * * * * \I3: relay -toggle; ftp -host "192.168.0.5" -user "guest" -pass "guest" -fmt sif -filename james/I3/test$h$m$s_$r.jpg -interval 100 -duration 1000 -cam 1; % #Event4 #buffer & motion initial at event start * * * * * RUN: buffer_init -cam 1 -fmt sif -imagenum 50; buffer_init -cam 2 -fmt sif -imagenum 20; buffer_start -cam 1 -fmt sif -interval 25 -duration -1; buffer_start -cam 2 -fmt sif -interval 25 -duration -1; motion_init -cam 1 -sensitivity 5; motion_init -cam 2 -sensitivity 5; motion_start -cam 1 -interval 100 -holdtime 1000; motion_start -cam 2 -interval 100; % #Event5 #GPIN2 rising to FTP all buffer * * * * * /I2: ftp -host 192.168.0.5 -user guest -pass guest -fmt sif -filename james/I2/test$h$m$s_$r.jpg -cam 1 -buffer; % #Event6 #GPIN1 rising to SMTP with buffer with 10 attachment * * * * * /I1: mail -subject "Test pic from NetComm" -fmt sif -recipient test@NetComm.com -cam 1 -imagenum 10 -host 192.168.0.1 -buffer -filename Mail$h$m$s_$r.jpg -msgbody "Hi This is Test\n\nABCDE\n\nFGHIJ "; % #Event7 #Motion Detect(SRC1) to Toggle the Relay and FTP * * * * * M1: relay -toggle; ftp -host 192.168.0.5 -user guest -pass guest -fmt sif -filename test/motion1/test$h$m$s_$r.jpg -interval 100 -duration 1000 -cam 1; http -id 0 -buffer -delay 0; http -id 1 -interval 10 -duration 500; % #Event8 #Motion Detect(SRC2) to FTP ring buffer * * * * * M2: relay -toggle; ftp -host 192.168.0.5 -user guest -pass guest -fmt sif -filename test/motion2/test$h$m$s_$r.jpg -cam 2 -buffer; % #Event9 #CGI trig1 to send string to com1 * * * * * G1: serial -comport 1 -message "Hi a message send which trig from CGI G1\n"; % #Event10 #CGI trig2 to log string to log.txt * * * * * G2: log -filename log.txt -message "Hi a log message send which trig from CGI G2\n"; % #Event11 #CGI trig3 to reboot system * * * * * G3: reset -flag 1; % #Event12 #Network Link lose to toggle relay #* * * * * \N: #relay -toggle; #%

To retrieve the buffer image when motion detected (pre alarm) using Netscape as below:

<html> <head> <title>Live Video Demo for Netscape</title> </head> <body> <H2>NetComm4000 Live Video Demo for Netscape(Server Push)</H2> <IMG SRC="http://192.168.0.101/getimage?camera=1&fmt=sif&id=0"> </body> </html>

To retrieve the current image when motion detected (postalarm). 
Note that due to we set the duration as 5 seconds, so the images will continue to feed out for 5 seconds when motion detected.

<html> <head> <title>Live Video Demo for Netscape</title> </head> <body> <H2>NetComm4000 Live Video Demo for Netscape(Server Push)</H2> <IMG SRC="http://192.168.0.101/getimage?camera=1&fmt=sif&id=1"> </body> </html>