NETCOMM.

NETCOMM Camera API, HTTP - Interface Specification

Firmware Version 2.20

Nov.14th.2002

Table of Contents

1.	Introduction	5

1.1.	Firmware versions	5

1.2.	Product-specific functionality	5

2.	DEFINITIONS	6

2.1.	General notations	6

2.1.1.	General abbreviations	6

2.2.	Style convention	6

2.2.1.	General CGI URL syntax and parameters	6

3.	INTERFACE SPECIFICATION	8

3.1.	Server URL syntax and Naming conventions	8

3.1.1.	HTTP status codes	8

4.	NetComm CGI requests	9

4.1.	Image and video request URLs	9

4.1.1.	JPEG image (snapshot)	9

4.1.2.	MJPEG video (server-push)	10

4.2.	CGI requests	13

4.2.1.	Clock adjustment	13

4.2.2.	Serial Port configuration	15

4.2.3.	Serial Port Transmission	17

4.2.4.	Server Device Configuration	17

4.2.5.	Network Configuration	19

4.2.6.	User configuration	21

4.2.7.	Video configuration	23

4.2.8.	PTZ configuration	25

4.2.9.	System Functions	27

4.2.10.	Video quality adjustment	28

4.2.11.	Motion detection configuration	30

4.2.12.	GPIO functions	32

4.2.13.	CCD module configuration	33

4.2.14.	CCD module configuration for NetComm 2XX series	34

4.2.15.	Event Script	36

4.2.16.	NetComm DDNS CGI	39

Document History

| Version | Date | Comment |
| --- | --- | --- |
| 1.16 | 2002-Feb-21 | Initial version. Support firmware since version 1.16 |
| 2.02 | 2002-Jun-12 | Remove event function API. Support firmware since version 2.02 |
| 2.10 | 2002-July-25 | Support firmware version 2.10 |
| 2.14 | 2002-Sep-02 | Correct CGI getimage function framedelay. Refer to MJPEG video (server-push) Add framedelay in camctrl CGI. Refer to Video quality adjustment |
| 2.18 | 2002-Oct-02 | Add Serial port transmission Serial Port Transmission |
| 2.19 | 2002-Nov-02 | Add NetComm DDNS CGI |
| 2.20 | 2002-Nov-14 | Add Additional Quality Adjustment for NetComm 205 |

# 1.   Introduction

This document specifies the external HTTP-based application-programming interface of the NetComm IP Camera (NetComm IP CAMERA herein and after).

The HTTP-based camera interface provides the functionality to request single and multi-part images, to control camera functions (PTZ, output relay etc.) and to get and set internal parameter values. The image and CGI-requests are handled by the built in Web server of the camera.

## 1.1. Firmware versions

The support for this stated HTTP API is highly dependent on product and release. Please refer to the actual product for corresponding information.

## 1.2. Product-specific functionality

Some of the functions described in this specification may not be implemented in every camera product, and the set of the CGI parameters and actual parameter values may differ among different camera products. Differences and deviations from this specification are described in the product specific API specifications.

# 2.   DEFINITIONS

This section contains information about general usages of this document.

## 2.1. General notations

### 2.1.1.       General abbreviations

| CGI | Common Gateway Interface – a standardized way to communicate between a client (e.g., a web browser) and a server (e.g., a web server). |
| --- | --- |
| N/A | Not applicable – a feature/parameter/value is useless in a specific task. |

## 2.2. Style convention

In URL syntax and in descriptions of CGI parameters, text in italic within angle brackets denotes that is to be replaced with either a value or a string. When replacing the text string, the angle brackets shall also be replaced. An example, denoted with <serverIP> in the URL, of this is the description of the URL is replaced with the string webcam.NetComm.com.

URL syntax’ is written with the “Syntax:” a word in bold face followed by a short explanation and then a box with the referred syntax, as seen below.

Syntax: this is a typical URL syntax

Http://<IpcameraIP>/images/NetComm.jpg

The name of the server is written as <IpcameraIP >. This intends to be replaced with the name of the actual server, which can be a name, e.g., “webcam” or “webcam.NetComm.com” or the associated IP number for the server, e.g., 192.168.0.200.

Description of returned data is written with “Return:” a word in a bold face followed by a short description and then the returned data in a box, as seen below.

Return: general web server returns

HTTP/1.0 <HTTP code> <HTTP text> \r\n

This is an HTTP data format. All data returned as HTTP format is line-separated with a Carriage Return and Line Feed (CRLF) printed as \r\n

Description of example is written with “Example:” a word in a bold face followed by a short description and a light gray box with the example, as seen below.

Example: request an image named “NetComm.jpg”.

http://webcam.netcomm.com/images/NetComm.jpg

### 2.2.1.       General CGI URL syntax and parameters

When the CGI request includes internal camera parameters, the internal parameters must be written exactly as they are named in the camera.

Syntax: general CGI request

http://< serverIP >/<cgi>[?<parameter>=<value>[?<parameter>=<value>…]]

Example: setting PTZ parameters

http://192.168.0.200/control?camid=1&pan=30

# 3.   INTERFACE SPECIFICATION

## 3.1. Server URL syntax and Naming conventions

### 3.1.1.       HTTP status codes

The built-in Web server uses the standard HTTP status codes.

Return: general web server returns:

HTTP/1.0 <HTTP code> <HTTP text> \r\n

HTTP code and text meanings are as the followings:

| HTTP code | HTTP text | Description |
| --- | --- | --- |
| 200 | OK | The request has succeeded. |
| 204 | No Content | Server has received the request but there is no information to feedback, and the client should stay in the same document view. This is mainly to allow inputting scripts without changing the document at the same time. |
| 400 | Bad Request | The request had bad syntax or was inherently impossible to be satisfied. |
| 401 | Unauthorized | The parameter to this message gives a specification of authorization schemes that are acceptable. The client should retry the request with a suitable Authorization header. |
| 403 | Forbidden | The request is forbidden by something. Authorization will be helpless. |
| 404 | Not Found | The server has not found anything matching the given URL. |

Example: request the unauthorized page.

HTTP/1.0 401 Unauthorized\r\n

# 4.   NetComm IP CAMERA CGI requests

## 4.1. Image and video request URLs

NetComm IP CAMERA provides two different ways to request images--- one is snapshot (JPEG); the other is server-push (MJPEG). Snapshot is the simplest way and natively supported by all browsers; and server-push is powerful and more efficient for network video streaming and natively supported by Netscape tm, but for some of browsers such as Explorer tm, need such as JAVA or ActiveX plug-in to achieve the server-push streaming mechanism.

The two different methods to make video request are explained as below,

Snapshot:

- 1.           Client connects to server, Server will accept the connection.

- 2.           Client sends snapshot image request to server.

- 3.           Server accepts the request with snapshot content type.

- 4.           Server sends one image to client.

- 5.           Client receives one image.

- 6.           Server actively closes the connection.

- 7.           Go back to step 1 if client needs more video.

Server-push:

- 1.           Client connects to server, Sever will accept the connection.

- 2.           Client sends server-push image request to server.

- 3.           Server accepts the request with server-push content type.

- 4.           Server sends one image to client with server-push boundary.

- 5.           Client receives one image.

- 6.           If more video needed, go back to step 4; if no video needed, continue step 7,

- 7.           Client actively closes the connection.

Please note there is no need to make request every time for server-push method, because it will reduce network traffic and eliminate socket re-creation, so it’s more efficient.

To get a snapshot JPEG image or a server-push Multipart JPEG image stream (MJPEG) using the following URLs.

### 4.1.1.       JPEG image (snapshot)

When a jpeg image is requested, the server either returns either the specified JPEG image file or an image with an error (No Video | Not Permission | Not available) .

Syntax: request JPEG image

http://< serverIP >/images<camid><image size>

With the following parameters:

| Parameter | Values | Description |
| --- | --- | --- |
| <camid> | 1~4 | Select the input camera source. This is dependent on product. |
| <image size> | “qsif”, “sif”, “full” | The returned jpeg image size. 176 by 112 in NTSC and 176 by 144 in PAL. 352 by 240 in NTSC and 352 by 288 in PAL. 704 by 480 in NTSC and 704 by 576 in PAL. |

Example: request JPEG image from input source 1 with sif size

http://192.168.0.200/images1sif

Return: requested JPEG image

HTTP/1.0 200 OK\r\n

Content-Type: image/jpeg\r\n

[ Content-Length: <image size>\r\n ]

[Authorization: <authorization information>\r\n ]

\r\n

<JPEG image data>\r\n

An optional field “Content-Length” header entry specifying the image size in bytes <image size> may also be included if the camera is configured to include it. Another optional field “Content-Auth” is followed by authorization specific data <authorization information>, e.g., in which encryption method is used.

Example: requested JPEG image

HTTP/1.0 200 OK\r\n

Content-Type: image/jpeg\r\n

Content-Length: 8567\r\n

Authorization: Basic cG14b3JkoJeyNjA4Nzk3\r\n

\r\n

<JPEG image data which start with 0xffd8 and end with 0xffd9>\r\n

### 4.1.2.       MJPEG video (server-push)

When an MJPG video is requested, the server returns either continuous flow of jpeg images or an image with an error (No Video | Not Permission | Not available) . The content type is “multipart/x-mixed-replace” and each image ends with a boundary string <boundary>. The returned image and HTTP data are equal to the request for a single JPEG image.

Syntax: request Multipart JPEG image

http://<serverIP>/getimage?camera=<camera id>&fmt=<image size>&delay=<delay>&id=<script ID>

With the following parameters:

| Parameter | Values | Description |
| --- | --- | --- |
| <camera id> | 1,2,3,4 | Select video source; omitted default is 1. |
| <iamge size> | “qsif”, “sif”, “full” | Specify MJPEG image size; omitted default is SIF. 176 by 112 in NTSC and 176 by 144 in PAL. 352 by 240 in NTSC and 352 by 288 in PAL. 704 by 480 in NTSC and 704 by 576 in PAL. |
| <delay> | 0~(232-1) | Specify delay between frames, unit in 10ms, omitted default is 0. It’s useful for network bandwidth control. |
| <script ID> | 0~9 | Specify http action node identification, and omitted default is to send continuous MJPEG images. If there is any matching id in programming script and the script is started, the MJPEG stream will pause until http action node triggered. Please refer to “Event Script Programmers Guide” for detail. |

Example: request JPEG image stream from the 2nd camera with sif resolution

http://192.168.0.200/getimage?camera=2&fmt=sif

Return: requested Multipart JPEG image

HTTP/1.0 200 OK\r\n

Content-Type: multipart/x-mixed-replace;boundary=--<boundary>\r\n

\r\n

--<boundary>\n

<image>

--<boundary>\n

<image>

…

where the <boundary> field in NetComm IP CAMERA is

<myboundary>\n

and the returned <image> field is

Content-Type: image/jpeg\n

Content-Length: <jpeg image size> Stamp:<YYYYMMDD 00HHmmss TK SSSSSSSS>\n\n

<jpeg image data>

Where

Stamp: Time stamp, of which format is with “Date”, “Time”, “Tick”, and “Sequence-number”
 		
Date:		Field 			Bits		Example
 		Year, A.D. 		31-16	07d2=2002 AD
 		Month (1~12) 		15-8		04=Apr
 		Day (1~31) 		7-0		01=First
 		
Time:	Field 			Bits
 		Hour (0~23) 		31-16	0011=17 hr
 		Minute (0~59) 		15-8		36=54 min
 		Second (0~59) 		7-0		0e=14 sec
 		
Tick:		Field 			Bits
 		Ticks(0~99)		7-0 		09=90 ms from last sec.
Seq:		Field 			Bits
 		Seq (0~232-1)		31-0 	84b=2123 images since server start

Example: requested Multipart JPEG image

HTTP/1.0 200 Okay\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: multipart/x-mixed-replace;boundary=--myboundary\r\n\r\n

\n\n-- myboundary\n

Content-Type: image/jpeg\n

Content-Length: 008376 Stamp:07d20717 00111404 22 00000ec6\n\n

<jpeg image data which start with 0xffd8 and end with 0xffd9>

\n\n-- myboundary\n

Content-Type: image/jpeg\n

Content-Length: 008376 Stamp:07d20717 00111404 22 00000ec6\n\n

<jpeg image data which start with 0xffd8 and end with 0xffd9>

…

…

## 4.2. CGI requests

### 4.2.1.       Clock adjustment

Adjust or read the server clock.

#### 4.2.1.1.       Clock time request

Syntax: request time from the server

http://<serverIP>/getclock

Return: requested time

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: <content_length>\r\n

\r\n

hr=<hour>\n

min=<minute>\n

sec=<second>\n

mn=<month>\n

date=<date>\n

year=<year>\n

Example: request time from the server

http://192.168.0.200/getclock

Return: requested time from the server

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 42\r\n

\r\n

hr=1\n

min=20\n

sec=38\n

mn=1\n

date=26\n

year=2000\n

You can try the CGI function with the browser.

The source of the HTML page is on the notepad. It’s the return values that you request via the CGI command.

#### 4.2.1.2.       Clock time adjust

Syntax: set time for the server

http://<serverIP> /setclock?Time=<hour>:<minute>:<second>:&Date=<year>/<month>/<date>

| Parameter | Values | Description |
| --- | --- | --- |
| <hour> | 1~ 23 | Hour |
| <min> | 1~59 | Minute |
| <sec> | 1~59 | Second |
| <month> | 1~12 | Month |
| <date> | 1~30 | Date |
| <year> | 0000~9999 | Year |

Example: request time from the server

http://192.168.0.200/setclock?Time=13:31:30&Date=2000/1/26

### 4.2.2.       Serial Port configuration

#### 4.2.2.1.       Configuration setup

Syntax: set serial port configuration

http://<serverIP>/setserial[?<parameter>=<value>[&<parameter>=<value>…]]

with the following parameters and values:

| Parameter | Values | Description |
| --- | --- | --- |
| port=<int> | 1,2 | The COM port is selected with this parameter. |
| mode=<string> | “RS485” / “RS232” | The serial mode of selected COM port. |
| usage=<int> | 2 | Reserved. It must be 2. |
| baud=<int> | 300,600,1200,2400,4800,9600,38400 | The baud rate of baud rate. |
| data=<int> | 4~8 | Data bits. |
| parity=<int> | 0,1 | Set to 1 if Parity check is enabled; otherwise, 0 is disabled.. |
| stop=<int> | 1, 2 | Stop bits. |

Example: set the COM2 of server.

http://192.168.0.200/setserial?port=2&mode=RS485&usage=2&

baud=9600&data=8&parity=0&stop=1

#### 4.2.2.2.       Configuration query

Syntax: request serial port configuration

http://<serverIP>/getserial?port=<port_id>

with the following parameters and values:

| Parameter | Values | Description |
| --- | --- | --- |
| < port_id > | 1,2 | COM1 or COM2 |

Return: requested serial port configuration

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 124\r\n

\r\n

Port=<port>\n

Serial Mode=<serial mode>\n

Usage=<usage>\n

Baud Rate=<baud rate>\n

Data Bits=<data bits>\n

Parity=<parity check>\n

Stop Bits=<stop bits>\n

Example: request the serial port configuration

http://192.168.0.200/getserial?port=2

Return: requested serial port configuration

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: <content_length>\r\n

\r\n

Port=COM2\n

Serial Mode=RS485\n

Usage=Generic\n

Baud Rate=9600\n

Data Bits=8\n

Parity=None\n

Stop Bits=1\n

### 4.2.3.       Serial Port Transmission

Syntax: set serial port data send/receive

http://<serverIP>/serial[?<parameter>=<value>[&<parameter>=<value>…]]

As the following parameters and values:

| Parameter | Values | Description |
| --- | --- | --- |
| port=<int> | 1,2 | The COM port is selected with this parameter. |
| write=<string> | Hex string of ASCII code, e.g. write=414243 as “ABC” | The string going to send through RS232/485 port. |
| read=<int> | 0..128 | The string length expected to receive. |

Example: Send string “Hello” to com2.

http://192.168.0.200/serial?port=2&write=48656c6c6f

Example: Send string “Hello” and wait for 5 characters from com2.

http://192.168.0.200/serial?port=2&write=48656c6c6f&read=5

Note:

- 1.           The read function will response as #HexAscII# format, e.g. above example will reply #41#42#43#44#45# when receiving “ABCDE” from com2.

- 2.           The read function expect to receive in 500ms, if can’t receive specific length of string in this period, error will response, such as “Error 00000001”.

### 4.2.4.       Server Device Configuration

#### 4.2.4.1.       Server configuration setting

Syntax: set server configuration

http://<serverIP>/setserver[?<parameter>=<value>[&<parameter>=<value>…]]

with the following parameters and values:

| Parameter | Values | Description |
| --- | --- | --- |
| device name=<string> | <characters> | The server name. |
| owner name=<string> | <characters> | The owner name. |
| owner email=<string> | <characters> | The E-mail address of the owner. |
| configuring camera at least once=<int> | 0,1 | Deprecated. |
| MAC address=<string> | <characters> | The MAC address of the server. This is read-only. |
| logoEnable=<int> | 0,1 | Shows/hides the logo. 0=hide, 1=show. |
| logo file name=<string> | <characters> | The file name of the logo image. |
| software Version=<string> | <characters> | The firmware version. |
| Revision=<string> | <characters> | Revision date. This is read-only. |
| Model=<int> | 1,2 | 1 for Canonical use, 2 for Generic use |
| Language=<int> | 0,1 | Select UI language. 0=English, 1=Traditional Chinese. |
| javamode=<int> | 0,1 | 0=Browsing live video using ActiveX. (default) 1=Browsing live video using Java Applet. |
| framedelay =<int> | 0~65535 | It’s a global frame delay setting. There’s also additional option of image to request CGI to achieve individual channel delay control. e.g. http://IP/getimage?camera=1&fmt=sif&framedelay={int}. 0 = No frame delay. (default) Note: If image request got framedelay parameter, it would override global setting. Also if there got framedelay parameter in Event system (HTTP action), it would override image request setting, so the priority of framedelay is Event>ImageRequest>Global Setting. |
| DhcpEnable =<int> | 0,1 | Set the server using DHCP to get dynamic IP Address. 0=Disable, 1=Enable. |

Example: set the server name to webcam1; owner to NetComm Co; and owner E-mail to service@NetComm.com, enable to display logo, set the file name of logo image to NetComm.gif, and set to use English language.

http://192.168.0.200/setserver?device name=webcam1&owner name=NetComm Co&owner email=service@NetComm.com&logoEnable=1&logo file name=NetComm.gif&Language=0

#### 4.2.4.2.       Server configuration request

Syntax: request server configuration

http://<serverIP>/server

Example: request the server configuration

http://192.168.0.200/server

Return: requested server configurations

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 244\r\n

\r\n

device name=webcam1\n

owner name=NetComm Co\n

owner email=service@domain.com\n

configuring camera at least once=1\n

MAC address=00-04-29-00-81-e0\n

logoEnable=1\n

logo file name=NetComm.gif\n

Software Version=1.15\n

Revision=0123.1630\n

Model=0\n

Language=0\n

javamode=0\n

framedelay=0\n

DhcpEnable=0\n

### 4.2.5.       Network Configuration

#### 4.2.5.1.       Network configuration setting

Syntax: set network configuration

http://<serverIP>/setnetwork[?<parameter>=<value>[&<parameter>=<value>…]]

with the following parameters and values:

| Parameter | Values | Description |
| --- | --- | --- |
| save=<int> | 0,1 | Save the configurations to EEPROM. 0=no, 1=yes. |
| IP address=<string> | <characters> | IP address of the server. |
| subnet mask=<string> | <characters> | Subnet mask for the server domain. |
| web server IP=<string> | <characters> | Deprecated. |
| web server name=<string> | <characters> | Deprecated. |
| dns1=<string> | <characters> | The 1st DNS server. |
| dns2=<string> | <characters> | The 2nd DNS server |
| dns3=<string> | <characters> | The 3rd DNS server |
| gateway=<string> | <characters> | The gateway of the domain. |
| http_port=<int> | 1~65535 | The Http connection port number. |

Example: set the server IP address to 192.168.0.129, subnet mask to 255.255.255.0, the 1st DNS server to 192.168.0.1, 2nd DNS server to 0.0.0.0, 3rd DNS server to 0.0.0.0, gateway to 192.168.0.254, and the Http connection port to 3080.

http://192.168.0.200/setnetwork?save=1&IP address=192.168.0.200&subnet mask=255.255.255.0

&dns1=192.168.0.1&dns2=0.0.0.0&dns3=0.0.0.0&gateway=192.168.0.254&http_port=3080

#### 4.2.5.2.       Network configuration request

Syntax: get network configuration

http://<serverIP>/network

Example: request the network configuration

http://192.168.0.200/network

Return: requested network configurations

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 124\r\n

\r\n

IP address=192.168.0.200\n

subnet mask=255.255.255.0\n

web server IP=236.160.238.0\n

web server name=www.NetComm.com\n

dns1=192.168.0.1\n

dns2=0.0.0.0\n

dns3=0.0.0.0\n

gateway=192.168.0.254\n

http_port=3080\n

### 4.2.6.       User configuration

#### 4.2.6.1.       User configuration setting

Syntax: set user configuration

http://<serverIP>/setusers[?<parameter>=<value>[&<parameter>=<value>…]]

with the following parameters and values:

| Parameter | Values | Description |
| --- | --- | --- |
| tag=<int> | 1,2,3,4 | Command type. 1=add, 2=modify, 3=delete, 4=who am I. |
| name=<string> | <characters> | User name. |
| password=<string> | <characters> | Password. |
| priority=<int> | 0,..,100 | Deprecated. |
| admin checked=<int> | <characters> | Administration Permission. 0=disable, 1=enable. |
| see image=<int> | 0,1 | Deprecated. |
| see video=<int> | 0,1 | Deprecated. |
| change param=<int> | 0,1 | Permission to change the Image type (Full, SIF, or QSIF) 0=disable, 1=enable. |
| see source1=<int> | 0,1 | Permission to see Video source 1. 0=disable, 1=enable. |
| see source2=<int> | 0,1 | Permission to see Video source 2. 0=disable, 1=enable. |
| see source3=<int> | 0,1 | Permission to see Video source 3. 0=disable, 1=enable. |
| see source4=<int> | 0,1 | Permission to see Video source 4. 0=disable, 1=enable. |
| see source_all=<int> | 0,1 | Permission to see the 4-split Video. 0=disable, 1=enable. |

Example: Add a user with name=admin and password=admin. And set this user with administration permission, change image type permission, see video 1 to 4, and see 4-split video permsiion.

http://192.168.0.200/setusers?tag=1&name=admin&password=admin&admin checked=1&change param=1&see source1=1&see source2=1&see source3=1&see source4=1&see source_all=1

#### 4.2.6.2.       User configuration request

Syntax: get user configuration

http://<serverIP>/users

Example: request the user configuration

http://192.168.0.200/users

Return: requested user configurations

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 326\r\n

\r\n

amount=2\n

name=admin\n

password=admin\n

priority=0\n

admin checked=1\n

see image=0\n

see video=0\n

change param=1\n

see source1=1\n

see source2=1\n

see source3=1\n

see source4=1\n

see source_all=1\n

name=guest\n

password=guest\n

priority=0\n

admin checked=0\n

see image=0\n

see video=0\n

change param=1\n

see source1=1\n

see source2=1\n

see source3=1\n

see source4=1\n

see source_all=1\n

### 4.2.7.       Video configuration

#### 4.2.7.1.       Videoconfiguration settings

Syntax: set user configuration

http://<serverIP>/setvideo[?<parameter>=<value>[&<parameter>=<value>…]]

with the following parameters and values:

| Parameter | Values | Description |
| --- | --- | --- |
| save=<int> | 0,1 | Save the configurations to EEPROM. 0=no, 1=yes. |
| VideoID=<int> | 1,..,5 | Specify which video channel will be set. 1=video1, 2=video2, 3=video3, 4=video4, 5=all the video channels. |
| JPEG Quality =<int> | 1,..,30 | Lower value means Higher Quality and smaller size. |
| BitmapEnable=<int> | 0,1 | Show / hide bitmap display. 0=hide, 1=show. |
| BitmapX=<int> | 0,..,703 | The X coordinate of the location of bitmap (valid value depends on the image size) |
| BitmapY=<int> | 0,..,479 | The Y coordinate of the location of bitmap (valid value depends on the image size) |
| BitmapTime=<int> | 0,1 | Show / hide time on bitmap. |
| BitmapTextEnable=<int> | 0,1 | Show / hide text on bitmap. |
| BitmapText=<string> | <characters> | Text to show on the bitmap. If the text = “null”, it will not be shown. <characters>,the characters must in the subset: {{0,..,9},{a,..,z},{A,..,Z}}. |
| ptz_model=<int> | 0,..,5 | Specify what kind of ptz camera is conntectd on the channel. 0=none, 1=sony D30, 2=Lilin series, 3=VC2000, 4=Kalatel series, 5=Dynacolor serise. |

Example: set the video channel 1 to display time on the bitmap at position (5,5), and the Jpeg compression level to 20. The connected camera is Lilin series.

http://192.168.0.200/setvideo?save=1&VideoID=1&BitmapEnable=1&Bitmap Time=1

&BitmapX=0& BitmapY=0&ptz_model=2

#### 4.2.7.2.       Video configuration request

Syntax: get video configuration

http://<serverIP>/video

Example: request the user configuration

http://192.168.0.200/video

Return: requested video configurations

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 326\r\n

\r\n

amount=4\n

JPEG Quality=20\n

BitmapEnable=1\n

BitmapX=5\n

BitmapY=5\n

BitmapTime=1\n

BitmapTextEnable=0\n

BitmapText=null\n

ptz_model=2\n

JPEG Quality=16\n

BitmapEnable=0\n

BitmapX=0\n

BitmapY=0\n

BitmapTime=1\n

BitmapTextEnable=0\n

BitmapText=null\n

ptz_model=0\n

JPEG Quality=16\n

BitmapEnable=0\n

BitmapX=0\n

BitmapY=0\n

BitmapTime=1\n

BitmapTextEnable=0\n

BitmapText=null\n

ptz_model=0\n

JPEG Quality=16\n

BitmapEnable=0\n

BitmapX=0\n

BitmapY=0\n

BitmapTime=1\n

BitmapTextEnable=0\n

BitmapText=null\n

ptz_model=0\n

In here, the amount=4 means there are 4 video channels connected on the server. So there will be four parts of the video configuration to be continually sent from the server.

### 4.2.8.       PTZ configuration

#### 4.2.8.1.       PTZ commands

Syntax: send the PTZ command

http://<serverIP>/control[?<parameter>=<value>[&<parameter>=<value>…]]

with the following parameters and values:

| Parameter | Values | Description |
| --- | --- | --- |
| camid=<int> | 1~4 | Camera Number for Pan, Tilt or Zoom |
| pan=<int> | 1~100 | Parameter of Pan control. This is only for Sony D30. Default value is 50. |
| tilt=<int> | 1~100 | Parameter of Tilt control. This is only for Sony D30. Default value is 50. |
| zoom=<int> | 1~100 | Parameter of Zoom control. This is only for Sony D30. Default value is 50. |
| rpan=<int> | -10~10 | Pan Relatively (Positive Value means Pan Right) |
| rtilt=<int> | -10~10 | Tilt Relatively (Positive Value means Tilt Up) |
| rzoom=<int> | -10~10 | Zoom Relatively (Positive Value means Zoom In) |

Example: set the camera 1 to pan right with speed 7.

http://192.168.0.200/control?camid=1&rpan=7

Example: set the camera 1 to pan to position 33. (Sony D30 only)

http://192.168.0.200/control?camid=1&pan=33

#### 4.2.8.2.       PTZ preset setting

Syntax: send the PTZ preset command

http://<serverIP>/ptzpreset[?<parameter>=<value>[&<parameter>=<value>…]]

with the following parameters and values:

| Parameter | Values | Description |
| --- | --- | --- |
| camid=<int> | 1~4 | Camera number for preset settings. |
| goto_preset=<int> | 0,..,19 | Goto a preset position according to the preset number. |
| set_preset=<int> | 0,..,19 | Assign a preset number to a preset position. |
| set_presetname=<string> | <characters> | Assign a preset name to a preset position. |
| rem_preset =<int> | 0,..,19 | Remove a preset name according the preset number. |

Example: set camera 1 to go to preset point 3.

http://192.168.0.200/ptzpreset?camid=1&gotopreset=3

Example: set the name “door” to the preset point 11.

http://192.168.0.200/ptzpreset?camid=1&gotopreset=11

http://192.168.0.200/ptzpreset?camid=1&set_preset=11&set_presetname=door

Example: remove a preset name according the preset point 5.

http://192.168.0.200/ptzpreset?camid=1&rem_preset=5

#### 4.2.8.3.       PTZ preset setting request

Syntax: request the PTZ preset settings

http://<serverIP>/ptzpreset

Example: request the PTZ preset settings

http://192.168.0.200/ptzpreset

Return: requested PTZ preset settings

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 74\r\n

\r\n

amount=4\n

1=window\n

3=door\n

4=chair\n

8=table\n

### 4.2.9.       System Functions

#### 4.2.9.1.       Reboot server

Syntax: reboot the server

http://<serverIP>/control?reboot=1

Example: request the server to reboot.

http://192.168.0.200/reboot

#### 4.2.9.2.       Write configurations permanently

Syntax: write configurations to EEPROM

http://<serverIP>/eepromsave

Example: request the server to write configurations to EEPROM.

http://192.168.0.200/eepromsave

#### 4.2.9.3.       Factory default

Syntax: restore configurations to factory default.

http://<serverIP>/factorydefault

Example: request the server to load factory default.

http://192.168.0.200/factorydefault

### 4.2.10.    Video quality adjustment

Syntax: Configure / Get video quality / parameters for individual source.

http://<serverIP>/camctrl[?<parameter>=<value>[&<parameter>=<value>…]]

With the following parameters and values:

| Parameter | Values | Description |
| --- | --- | --- |
| cmd=<string> | set,get,camstat | It is necessary to choose what kind of command. ‘set’=set the video quality settings. ‘get’=get the video quality settings. ‘camstat’=return the format of the connected video. (0=No Video, 1=NTSC, 2=SECAM, 3=PAL) |
| camid=<int> | 1~4 | Camera Source to be adjusted. |
| contrast=<int> | -100~100 | Contrast level adjustment. Default value is 0. |
| bright=<int> | -100~100 | Brightness level adjustment. Default value is 0. |
| hue=<int> | -100~100 | Hue level adjustment. Default value is 0. |
| saturation=<int> | -100~100 | Saturation level adjustment. Default value is 0. |
| framedelay=<int> | 0~(232-1) | Set delay time between frames, unit in 10ms, Default value is 0. |
| ptzid=<int> | 1~255 | Control ID of PTZ device. By default, video-1 will be assigned to ptz-1, video-2 to ptz-2, etc. |

Example: set the video quality of video channel 1.to default values.

http://192.168.0.200/camctrl?cmd=set&camid=1&contrast=0&bright=0&hue=0&saturation=0

Example: request the video quality of video channel 1.

http://192.168.0.200/camctrl?cmd=get&camid=1

Return: requested PTZ preset settings

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 114\r\n

\r\n

contrast=0\n

bright=0\n

hue=0\n

saturation=0\n

sharp=0\n

pan=50\n

tilt=50\n

zoom=50\n

ptzid=1\n

Example: request the video format

http://192.168.0.200/camctrl?cmd=camstat

Return: requested video format

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 68\r\n

\r\n

Cam1=1\n

Cam2=1\n

Cam3=1\n

Cam4=0\n

#### 4.2.10.1.    Additional Quality Adjustment for NetComm 205

For NetComm 205, more properties could be accessed through same CGI “camctrl”, which is listed as below:

| Parameter | Values | Description |
| --- | --- | --- |
| sharp=<int> | -100~100 | Sharpness adjustment. Default value is 0. |
| exposure=<int> | -100~100 | Control the electronic shutter manually, valid only when “autoexposure” below set to off. Default value is 0. |
| backlight=<int> | 0~1 | Set back light compensation to off (0), or on (1). Default value is 0. |
| autobright=<int> | 0~1 | Set brightness controll manually (0), or auto (1). Default value is 1. |
| autoexposure=<int> | 0~1 | Set electronic shutter control manually (0), or auto (1). Default value is 1. |
| flicker=<int> | 0~1 | Set flicker control to 50Hz(0), or 60Hz(1). Default value is 1. |
| lightcondition=<int> | 0~2 | Set proper light condition to Tungsten (0), or fluorescent (1), or Outdoor (2). Default value is 1. |

Example: set the video quality of NetComm 205 to manual brightness and increase 50% of brightness.

http://192.168.0.200/camctrl?cmd=set&camid=1&autobright=0&bright=50

### 4.2.11.    Motion detection configuration

Syntax: motion detection setting

http://<serverIP>/motion[?<parameter>=<value>[&<parameter>=<value>…]]

with the following parameters and values:

| Parameter | Values | Description |
| --- | --- | --- |
| cmd=<string> | get,set | It is necessary to choose what kind of command. ‘get’=request the motion detection settings. ‘set’=set the motion detection settings. |
| mdEv=<int> | 0,1 | Motion detection enable or not. 0=disable, 1=enable. |
| mdSen=<int> | 1~30 | Motion detection sensitivity. Larger value means less sensitive. Default value is 5. |
| mdFreq=<int> | 1~100 | Polling Frequency in 1/10 second. Default value is 5. |
| mdX0=<int> | 0~10 | Leftmost Coordinates of Grid Number. |
| mdX1=<int> | 0~10 | Rightmost Coordinates of Grid Number. |
| mdY0=<int> | 0~6 | Top Coordinates of Grid Number. |
| mdY1=<int> | 0~6 | Bottom Coordinates of Grid Number. |
| mdBlock=<int>[,<int>,..] | 0,..,76 | A series of Blocks to detect motion if using GRID method. |

Example: enable the motion detetcion function and set the sensitivity to 5 and frequency to 0.5 second for video channel 2. And set the motion detection area to block 1, 12, and 13.

http://192.168.0.200/motion? cmd=set&camid=2&mdEv=1&mdSen=5&mdFreq=5&mdBlock=1v12v13v

The mdBlock=1v12v13v parameter means the motion detection areas are block 1, block 12, and block 13, as the red areas in the following map.

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 |
| 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 |
| 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 |
| 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 |
| 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 |

Example: request the motion detection configuration

http://192.168.0.200/motion?cmd=get&camid=2

Return: requested motion detection configuration

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 48\r\n

\r\n

mdEv=1\n

mdSen=5\n

mdFreq=5\n

mdBlock=1:12:13\n

### 4.2.12.    GPIO functions

#### 4.2.12.1.   GPIO input configuration request

Syntax: request GPIO input configuration

http://<serverIP>/io?input=check

Example: request GPIO input configuration

http://192.168.0.200/io?input=check

Return: requested motion detection configuration

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 46\r\n

\r\n

Input1=1\n

Input2=1\n

Input3=1\n

Input4=1\n

In this example, there are four GPIO inputs on the server. If the GPIO input is low, the GPIO input=1. If the GPIO input is high, the GPIO input=0.

#### 4.2.12.2.   GPIO relay configuration request

Syntax: request GPIO relay configuration

http://<serverIP>/io?relay=check

Example: request GPIO relay configuration

http://192.168.0.200/io?relay=check

Return: requested the GPIO relay configuration

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 32\r\n

\r\n

Relay=0\n

If Relay=0, the GPIO relay is high. Else if the Relay=1, the GPIO relay is low.

### 4.2.13.    CCD module configuration

#### 4.2.13.1.   CCD module setting

Syntax: CCD module setting

http://<serverIP>/io[?<parameter>=<value>[&<parameter>=<value>…]]

with the following parameters and values:

| Parameter | Values | Description |
| --- | --- | --- |
| aes=<int> | 0,1 | Activate auto electronic shutter or not. 0=disable, 1=enable. |
| blc=<int> | 0,1 | Activate back light compensation or not. 0=disable, 1=enable. |
| day=<int> | 0,1 | Activate day night control or not. 0=disable, 1=enable. |

#### 4.2.13.2.   CCD module get

Syntax: request CCD module configuration

http://<serverIP>/io?ccd=check

Example: request the CCD module configuration

http://192.168.0.200/io?ccd=check

Return: requested CCD module configuration

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 32\r\n

\r\n

ms=0\n

aes=1\n

blc=0\n

day=0\n

DIP Activate, Internal CCD\n

### 4.2.14.    CCD module configuration for NetComm 2XX series

There are two output destinations for the control data from the CCD module AE: AGC gain (EVR), shutter speed (TG). The CCD module output states to these destinations for each operating mode are shown in the table below.

In here, we use a byte to represent the CCD module settings.

| Bits | Bits 7 | Bits 6 | Bits 5 | Bits 4 | Bits 3 | Bits 2 | Bits 1 | Bits 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Function | I/E Sync | AE / AESHUT mode |  |  |  | AGC | AWB |  |

Bit 1,0:

| Bit 1 | Bit 0 | AWB (Auto White Balance) mode |
| --- | --- | --- |
| 0 | 0 | ATW |
| 0 | 1 | INDOOR |
| 1 | 0 | PUSH |
| 1 | 1 | OUTDOOR |

Bit 2:

| Bit 2 | AGC (Auto Gain Control) mode |
| --- | --- |
| 0 | OFF |
| 1 | ON |

Bit 3 ~ 6:

| Bit 6 | Bit 5 | Bit 4 | Bit 3 | AE (Auto Electronic) mode |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | AE |
| 1 | 0 | 0 | 0 | AE flickerless |
| 0 | 1 | 0 | 0 | AE BLC off |
| 1 | 1 | 0 | 0 | AE flickerless BLC off |
| 0 | 0 | 1 | 0 | AE Mechanical iris |
| 1 | 0 | 1 | 0 | AE Mechanical iris Flickerless |
| 0 | 1 | 1 | 0 | AE Mechanical iris BLC off |
| 1 | 1 | 1 | 0 | AE Mechanical iris, BLC off, flickerless |
| 0 | 0 | 0 | 1 | AE-SHUT 1/60 (1/50) |
| 1 | 0 | 0 | 1 | AE-SHUT 1/100 (1/120) |
| 0 | 1 | 0 | 1 | AE-SHUT 1/250 |
| 1 | 1 | 0 | 1 | AE-SHUT 1/500 |
| 0 | 0 | 1 | 1 | AE-SHUT 1/1000 |
| 1 | 0 | 1 | 1 | AE-SHUT 1/2000 |
| 0 | 1 | 1 | 1 | AE-SHUT 1/4000 |
| 1 | 1 | 1 | 1 | AE-SHUT 1/100000 |

Bit 7:

| Bit 7 | I/E Sync |
| --- | --- |
| 0 | Internal signal |
| 1 | External signal |

To set the CCD module just combine the bits 0 to bits 7 to form a byte and send the following CGI function with this byte. On the other hand, you can also get the CCD settings in byte from the server and parsing it into bits to know the detailed configurations of the CCD module.

#### 4.2.14.1.   Set the CCD module

Syntax: Set the CCD module setting

http://<serverIP>/io?ccdcmd=setbyte&<parameter>=<value>

with the following parameters and values:

| Parameter | Values | Description |
| --- | --- | --- |
| ccdval=<int> | 0~255 | The CCD settings in byte value. |

Example: set the CCD module to push mode, no AGC, AE 1/250, and internal video.

http://192.168.0.200/io?ccdcmd=setbyte&ccdval=41

#### 4.2.14.2.   Get the CCD module status

Syntax: Get CCD module setting

http://<serverIP>/io?ccdcmd=getbyte

Return: requested CCD module configuration

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 32\r\n

\r\n

ccdret=41

### 4.2.15.    Event Script

The Event Script provides a flexible way for Administrator to configure NetComm  camera as automatic intelligent devices in applications. A comprehensive set of “trigger conditions” and “action commands” allowed fitting in various of applications without limitation. The more information about the Event Script, please see the Event Script Programmer’s Guide.

#### 4.2.15.1.   Get the event script code (text)

Syntax: get the event script from the IP Camera

http://<serverIP>/evscript?getscript

Return: requested event script

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 440\r\n

\r\n

# Mail Camera  image

# to jameslee@domain.com

# with  size, when

# Motion Detect on channel 1

* * * * * RUN:

motion_init

-cam 1

-sensitivity 5;

motion_start

-cam 1

-interval 50

-holdtime 50;

%

* * * * * M1:

relay -toggle;

mail -host 192.168.0.5

-recipient jameslee@domain.com

-filename james$h$m$s.jpg

-msgbody "test"

-returnaddr jameslee@domain.com

-subject "test"

-cam 1

-fmt sif

-imagenum 1;

%

#### 4.2.15.2.   Set event script status

Syntax: set the event script to the camera

http://<serverIP>/evscript?<parameter>=<value>

with the following parameters and values:

| Parameter | Values | Description |
| --- | --- | --- |
| setscript =<string> | <characters> | Send the script text to the server. |
| runcmd=<string> | “start”, “stop” | Set and get the event script startup type. “start”=start to run the event script. “stop”=stop running the event script. |
| setboot=<int> | 0,1 | Set the event script to start up when the system is start-up. |

Example: send the script text to the server with encode.

http://192.168.0.200/evscript?setscript=%23%20%4d%61%69%6c%20%43%61%6d%65%72%61%20%20%69%6d%61%67%65%0d%0a%23%20%74%6f%20%6a%61%6d%65%73%6c%65%65%0d%0a%20%2d%66%6d%74%20%73%69%66%0d%0a%20%2d%69%6d%61%67%65%6e%75%6d%20%31%3b%0d%0a%25%0d%0a

#### 4.2.15.3.   Check the event script status

To know if the event script is running or not.

Syntax: set the event script to the camera

http://<serverIP>/evscript?runcmd=check

Return: requested event script

HTTP/1.0 200 OK\r\n

Date: Thu, 01 Jan 1970 00:00:00 GMT\r\n

Connection: close\r\n

Content-Type: text/html\r\n

Content-Length: 33\r\n

\r\n

Parsing=0

RunStatus=0

BootFlag=0

### 4.2.16.    NetComm DDNS CGI

The NetComm DDNS (Dynamic Domain Name Service) is used when users want to access NetComm NC with an easy memorized name such as http://demo.ddns.NetComm.com instead of http://61.220.235.172.

This service could be useful when NetComm IP CAMERA is located behind Dial-up ADSL or IP sharing devices, which does not have fix IP address, then it’s impossible to reach NetComm NC from Internet.

The mechanism of NetComm DDNS service is described as below:

- 1.       When NetComm IP CAMERA enables the DDNS service, it will “register” to NetComm DDNS server with its information, such as server name to access, router virtual port number and updated frequency, etc.

- 2.       Then NetComm IP CAMERA automatically “update” to NetComm DDNS server by a fix frequency, so even IP is changed by ISP, the NetComm DDNS server still could get and update internal database.

Then, once users access from Internet with its register name, e.g. if registering with server name “demo” to DDNS server “ddns.NetComm.com”, NetComm NC could be accessed by http://demo.ddns.NetComm.com.

#### 4.2.16.1.   Get DDNS configurations

Syntax: get the event script from the camera

http://<serverIP>/ddns?cmd=get

Return: Query DDNS configurations

DDNS Configuration:

enable=0

ddnsaddr=ddns.NetComm.com

ddnsport=80

routerinport=8000

updatetime=10

#### 4.2.16.2.   Set DDNS configurations

Syntax: set the DDNS configurations

http://<serverIP>/ddns?cmd=set&<parameter>=<value>

As the following parameters and values:

| Parameter | Values | Description |
| --- | --- | --- |
| enable=<int> | 0,1 | 0:Disable, 1:Enable DDNS service. |
| ddnsaddr=<string> | Omit default: “ddns.NetComm.com” | Specifiy the DDNS server IP address. Could use domain name. |
| ddnsport=<int> | 0..65535, Omit default: 80 | Specifiy the DDNS server listen port. |
| routerinport=<int> | 0..65535, Omit default: Same as HTTP port of NetComm IP CAMERA | Specifiy the IP Sharing device’s virtual server listen port, for example, the IP sharing may listen to port 8000 to redirect to port 10000 of NetComm IP CAMERA, then should set this parameter with 8000. |
| updatetime=<int> | Unit by second. Omit default: 600 seconds (10 minutes) | Specifiy the automatic update frequency once start DDNS service. |

Example:

http://192.168.0.200/ddns?cmd=set&enable=1&ddnsaddr=ddns.NetComm.com&ddnsport=80&routerinport=8000&updatetime=600

Note: The server name to access is configured by CGI Server configuration setting.

Return: set the DDNS configurations

Succeeded [CGI fail/Already Registered]

If command fails, following hints may help to diagnostic:

- 1.       Confirm your Network setting is correct, including “Gateway IP” to reach out side world, “DNS1 IP” address should be public and reachable, and “HTTP port” should be configured the same with your IP sharing redirection port. Try to ping above IP address if reachable.

- 2.       The “routerinport” should be the same as router (IP Sharing) listen port.

- 3.       Try different server name, or add company prefix to avoid conflicting inside DDNS server database. E.g. company.user.ddns.NetComm.com.