Extandon XT1800 ADSL Modem Setup Guide

English Version, 15Jul05

© Extandon 2005

TABLE OF CONTENTS

Minimum system requirements	3

Installation of the Extandon XT1800 ADSL Modem (PPPoE, PPPoA, RFC1483)	3

Un-installation of the Extandon XT1800 ADSL Modem	3

Re-configuration of the Extandon XT1800 ADSL Modem	4

Up-gradation of the Extandon XT1800 ADSL Modem	4

Using the ADSL Analyzer	5

Instruction for Connecting to the Internet	7

Connecting to ISP using CD provided by the ISP	7

Windows XP – ISP connection: Setup for PPPoE	7

Windows XP – ISP connection: Setup for PPPoA	7

Windows XP – ISP connection: Setup for RFC1483	8

Windows 2000 – ISP connection: Setup for PPPoE	8

Windows 2000 – ISP connection wizard: Setup for PPPoA	8

Windows 2000 – ISP connection: Setup for RFC1483	9

Windows 98/Millenium – ISP connection: Setup for PPPoE	9

Windows 98/Millenium – ISP connection: Setup for PPPoA	9

Windows 98/Millenium – ISP connection: Setup for RFC1483	10

Trouble-shooting	11

## Minimum system requirements

The Extandon XT1800 ADSL Modem can be installed on a PC that meets the following minimum requirements

Processor: 		Intel P4/Celeron 2.0 GHz or equivalent.

Operating System:	Microsoft Windows 98 Second Edition, Windows Millennium, Windows 2000 or Windows XP.

## Installation of the Extandon XT1800 ADSL Modem (PPPoE, PPPoA, RFC1483)

- Power off the PC, unplug the power cord and open the cover. Install the Extandon XT1800 ADSL Modem card in an available PCI slot, close the cover and plug back the power card.

- Power on the PC and login as a User/Administrator. The Windows operating system will detect the presence of the Extandon XT1800 ADSL Modem and will pop up a “Found New Hardware Wizard” window. Press cancel to stop Windows install process.

- Run the program “Setup.exe” that is provided on the CD, to install the driver files.

- If the driver has undergone WHQL certification, it is automatically installed.  Directly proceed to Step - 8.

- Windows 2000 and Windows XP ONLY:

A “Software Installation” dialog box may appear multiple times. Please click “Continue Anyway”. The “Found New Hardware Wizard” will appear. Select “Install the software automatically”. Click “Next”. Click “Finish” to complete the driver installation.

- Windows 98/Millenium ONLY:

The “Add New Hardware Wizard” will appear. Click “Next”. Select “Search for the best driver for your device”. Click “Next”.

- Windows 98/Millenium ONLY:

Select the “Specify a location” checkbox. Make sure that the directory is INSTALLDIR\win98 for PPPoE/RFC1483 and INSTALLDIR\win98\pppoa for PPPoA. Click “Next”. Click “Next” again. Press “OK” when the “Copying files” dialog appears. The Windows 98/Millenium installation CD is required for additional Windows components to be installed. Click “Finish” to complete the driver installation.

- The setup program will install the driver and copy the driver files/utilities to INSTALLDIR and auto start the ADSL Analyzer application (DslGui.exe). A shortcut to the application is provided on the Desktop for convenience. A “network” connection for the ISP is also created.

- Reboot the PC, when requested.

- The “ADSL Analyzer” icon will be visible in the system tray. Double-click the icon to start the program. The modem is setup in the idle state. Press the “Connect” button to connect to the ISP. Press the “Start Modem” button if only a physical connection is required.

## Un-installation of the Extandon XT1800 ADSL Modem

- Double click the “ADSL Analyzer” icon in the system tray. If the connection is active, stop the modem using the “Stop modem” button on the “General” property page. Close the application.

- Windows 98/Millenium ONLY:

If the mode encapsulation is PPPoA, the Microsoft PPP over ATM adapter has to be manually removed. Right-click on “My Computer”, select “Properties”. Select “Device Manager”. Select “Network Adapters”. Select “PPP over ATM adapter”. Click “Remove”. Re-boot the PC.

- The package can be removed through the “Add/Remove programs” application in the “Control Panel”. Click “OK” when the “Save Configuration” dialog box occurs, if the current configuration settings need to be saved.

## Re-configuration of the Extandon XT1800 ADSL Modem

- Double click the “ADSL Analyzer” icon in the system tray. If the connection is active, stop the modem using the “Stop modem” button on the “General” property page.

- Select the “Configuration” page. Press the “Modify Config” button. In the pop-up dialog box, select and update the desired configuration change. Click “OK” to proceed with the modifications.

- If the “Encapsulation“change PPPoE <-> RFC1483 <-> PPPoA is desired, then the driver has to be re-installed. Click the “Change Driver” button on the “Configuration” page. Wait till the driver re-installation process is complete and the driver change is visible on the “Configuration” page.

- Reboot the PC.

## Up-gradation of the Extandon XT1800 ADSL Modem

Up-gradation of the package may be required if a new version of the installer package is provided.

- Uninstall the old package. Click “OK” when the “Save Configuration” dialog box occurs, if the current configuration settings need to be saved.

- Proceed with the installation of the new package.

## Using the ADSL Analyzer

The application is installed as a part of the installation package, and should be visible on the system tray.

To run the application, simply double-click the icon on the system tray. The application consists of a set of four property sheets, viz., “General”, “Statistics”, “Spectrum” and “Configuration”. Use the mouse to select the desired property sheet.

The “General” property sheet contains the following information.

Transmit rate:	Data rate (in Kbps) of the ADSL connection in the upstream direction.

Receive rate: 	Data rate (in Kbps) of the ADSL connection in the downstream direction.

ADSL mode: 	Mode of the connection, i.e., the standard governing the connection.

Encapsulation: 	Encapsulation used to exchange data over the connection.

Installer version: 	Version of the device driver package for the ADSL modem.

Server version: 	Version of the server modem, at the other end of the connection.

Connection status:	Current status of the higher layer connection

ADSL status: 		Current status of the physical layer connection.

Active time: 	Time for which the last connection was active in hh:mm:ss format.

The “General” property sheet also contains the following buttons.

Stop/Start Modem:	This button can be used to start and stop the modem. If the connection is active, stopping the modem will close the connection. Similarly, a connection can be re-established by starting the modem.

Connect/Disconnect: 	This button can be used to start a higher layer connection. If the ISP selection is configured, the modem will start the dial process. If not, a dialog box listing the phone book entries that use this modem is popped up. The user can select the desired phonebook entry to dial.

Generate Log: 	This button creates a log file containing parameters related to the last active connection. The file “modem.dat” is created in the “Install” directory. The file can be mailed to the support team, in the event of a connection-related problem.

A connection status icon is also provided. The icon changes colour depending on the status of the ADSL connection. The colour red indicates that the connection is inactive, yellow colour indicates that the connection is synchronizing and green colour indicates that the connection is active. The application can be minimized to the system tray, as shown in the figure below. The modem states is unchanged if the application is minimized or closed.

The “Statistics” property sheet contains the following information.

Bit error rate: 	Current bit error rate (bits in error in a million received bits) of the ADSL connection in the downstream direction.

Power: 	Average power spectral density in the upstream direction (in dBm/Hz)

SNR margin:	Signal to noise ratio (in dB) in the downstream direction.

Line attenuation:	Attenuation (in dB) of the signal in the downstream direction.

Performance margin:	Required signal to noise ratio (in dB) of the connection in the downstream direction.

The “Spectrum” property sheet contains transmit and receive bit loading across the bins.

The “Configuration” property sheet can be used to change ISP-related configuration parameters, switch the driver or uninstall the driver.

The “Configuration” property sheet contains the following information.

Encapsulation:		Encapsulation used to exchange data over the connection.

ADSL mode:	Mode of the connection, i.e., the standard governing the connection.

ISP connection:	Phone book entry corresponding to the ISP.

MAC address:	48 bit MAC (Ethernet) address of the modem.

VPI: 	The Virtual Path Identifier (VPI) used for the ATM connection.

VCI: 	The Virtual Circuit Identifier (VCI) used for the ATM connection.

Boot start:	0: Modem does not attempt a connection when the PC boots.

1: Modem attempts a physical layer connection when the PC boots.

2: Modem attempts an ISP connection when the PC boots.

Profile:	Special settings needed to work with some ISPs. This should be changed only if requested by the Support team.

Install status:	Current state of the driver installation process. This is especially useful when the driver is changed from PPPoE/RFC1483 to PPPoA and vice-versa.

The “Configuration” property sheet contains the following buttons.

Modify Config:	This button allows the user to change ISP-related configuration parameters specified above. The user can change the parameters to the desired values and then press “OK” to effect the changes. For the MAC address field, the upper 24 bits are fixed and the lower 24 bits can be modified. This field requires hexadecimal characters (0…9, A…F).

Save Profile:	This button allows the user to save all settings (called the “profile”) into a file. This can be retrieved at a later point.

Change Driver:	Three driver sets are provided, one for PPPoE connections, one for RFC1483 connections and the other for PPPoA connections. The user can switch between the PPPoE <-> RFC1483 <-> PPPoA.

## Instruction for Connecting to the Internet

During installation an ISP connection “XT1800 Modem Connection” is created.

- Double click the ADSL Analyzer icon on the tray.

- Click “connect”. This will automatically start the modem and initiate a network connection. Wait till the “Connection status” shows “Connected”.

- Check the internet connection using Internet Explorer or other applications.

The sections below are to be used ONLY if the user requires to manually create a connection.

## Connecting to ISP using CD provided by the ISP

For Internet Connection, install the connection applet using the ISP provided CD and select the “PCI modem” or “Other modem” option.    In case of problem using the ISP provided CD, use the native XP connection wizard to setup the PPPoE/RFC1483 or PPPoA connection as specified in the last section.

Note down the Protocol Encapsulation (RFC1483, PPPoE, PPPoA, etc.), Modulation Mode (Multimode, GDMT, etc.), VPI and VCI. Make sure the ADSL Analyzer configuration matches this.

## Windows XP – ISP connection: Setup for PPPoE

- Double click the “New connection wizard” to setup the ISP connection.

- Welcome panel appears – press Next.

- Select “Connect to the Internet” – press Next.

- Select “Set up my connection manually” – press Next.

- Select “Connect using a broadband that requires a user name and password” – press Next.

- Fill in your ISP name (or any name you like) – press Next.

- Fill in User name, Password provided by the ISP and confirm password – press Next.

- Press Finish to exit the wizard.

- The dialer screen will come up.

- Press “Dial” to dial out immediately, or “Cancel”.

- An icon with your ISP name would be created under Broadband.

This completes the ISP connection configuration process for PPPoE.

## Windows XP – ISP connection: Setup for PPPoA

- Double click the “new connection wizard” to setup the ISP connection.

- Welcome panel appears – press Next.

- Select “Connect to the network at my workplace” – press Next.

- Select “Dial-up connection” – press Next.

- Select “Extandon XT1800 ADSL Modem for PPPoATM” as the device – press Next.

- Write the ISP name (or any name you like) in the company Name – press Next.

- In phone number space, write pVPI,VCI (ex: p0,35) provided by ISP – press Next.

- Press Finish to exit the wizard.

This completes the ISP connection configuration process for PPPoA. The dialer screen will come up and can be used to dial out.

## Windows XP – ISP connection: Setup for RFC1483

- Double click the “My Network Places” icon. Click “View Network Connections”.

- Under “LAN or High-Speed Internet”, select the connection corresponding to the Extandon XT1800 ADSL Modem.

- Right-click on the connection and select “Properties”. Double-click on “Internet Protocol TCP/IP”.

- Select “Use the following IP address”. Enter the IP address, Sub-net Mask and Default Gateway. These should be provided by the ISP provider. If they are not available, select “Obtain an IP address automatically”.

- Select “Use the following DNS server addresses”. Enter the DNS server addresses. These should be provided by the ISP provider. If they are not available, select “Obtain DNS server address automatically”.

This completes the ISP connection configuration process for RFC1483. Start the modem using the ADSL Analyzer to activate the connection.

## Windows 2000 – ISP connection: Setup for PPPoE

- Double click the “new connection wizard” to setup the ISP connection.

- Welcome panel appears – press Next.

- Select “Dial-up to private network” – press Next.

- Select “PPP over Ethernet – Extandon XT1800 ADSL Modem for PPPoE (SW5610-0)” – press Next.

- Ignore “Phone number to dial” – press Next.

- Select “For all users” – press Next.

- Write the ISP name (or any name you like) – press Next.

- An icon with your ISP name would be created on the desktop.

This completes the ISP connection configuration process for PPPoE and RFC1483. The dialer screen will come up and can be used to dial out.

## Windows 2000 – ISP connection wizard: Setup for PPPoA

- Double click the “new connection wizard” to setup the ISP connection.

- Welcome panel appears – press Next.

- Select “Dial-up to private network” – press Next.

- Select “Extandon XT1800 ADSL Modem for PPPoATM” – press Next.

- Ignore “Phone number to dial” – press Next.

- Select “For all users” – press Next.

- Write the ISP name (or any name you like) – press Next.

- An icon with your ISP name would be created on the desktop.

This completes the ISP connection configuration process for PPPoA. The dialer screen will come up and can be used to dial out.

## Windows 2000 – ISP connection: Setup for RFC1483

- Double click the “My Network Places” icon. Click “Network and Dial-up Connections”.

- Under “LAN or High-Speed Internet”, select the connection corresponding to the Extandon XT1800 ADSL Modem.

- Right-click on the connection and select “Properties”. Double-click on “Internet Protocol TCP/IP”.

- Select “Use the following IP address”. Enter the IP address, Sub-net Mask and Default Gateway. These should be provided by the ISP provider. If they are not available, select “Obtain an IP address automatically”.

- Select “Use the following DNS server addresses”. Enter the DNS server addresses. These should be provided by the ISP provider. If they are not available, select “Obtain DNS server address automatically”.

This completes the ISP connection configuration process for RFC1483. Start the modem using the ADSL Analyzer to activate the connection.

## Windows 98/Millenium – ISP connection: Setup for PPPoE

- Double-click on the “My Computer” icon on the desktop. Select the “Dial-up Networking” folder – press Make New connection.

- Write the ISP name (or any name you like). Select “PPP over Ethernet Adapter-Line01” as the device.

- Enter 0 in the “Telephone Number”. This field is actually ignored – press Finish. This will create a new dial-up connection.

This completes the ISP connection configuration process for PPPoE. The connection created in the “Dial-up Networking” folder in the “Control Panel” window can be clicked to trigger the dial-up process. The dial-up process may indicate an error, until the physical layer is established and the icon turns green. Please ignore.

## Windows 98/Millenium – ISP connection: Setup for PPPoA

- Double-click on the “My Computer” icon on the desktop. Select the “Dial-up Networking” folder – press Make New connection.

- Write the ISP name (or any name you like). Select “PPP ATM Adapter” as the device – press Configure.

- Set the “Circuit Type” to “Permanent Virtual Circuit” with the VPI/VCI values set to the ones provided by the ISP. Set the “Framing option” to “Null encapsulation” – press OK.

- Enter 0 in the “Telephone Number”. This field is actually ignored – press Finish. This will create a new dial-up connection.

This completes the ISP connection configuration process for PPPoA. The connection created in the “Dial-up Networking” folder in the “Control Panel” window can be clicked to trigger the dial-up process. The dial-up process may indicate an error, until the physical layer is established and the icon turns green. Please ignore.

## Windows 98/Millenium – ISP connection: Setup for RFC1483

- Right click on the “Network Neighborhood” icon. Click “Properties”.

- Select “TCP/IP -> Extandon XT1800 ADSL Modem”. Click “Properties”.

- Select the “IP address” property sheet. Select “Specify an IP address”. Enter the IP address and Sub-net Mask. These should be provided by the ISP provider. If they are not available, select “Obtain an IP address automatically”.

- Select the “Gateway” property sheet. Enter the gateway address and press “Add”. This should be provided by the ISP provider. If this is not available, skip this step.

- Select the “DNS Configuration” property sheet. Select “Enable DNS”. Enter the DNS server address in the “DNS Server Search Order” and press “Add”.  This should be provided by the ISP provider. If this is not available, select “Disable DNS”.

This completes the ISP connection configuration process for RFC1483. Start the modem using the ADSL Analyzer to activate the connection.

## Trouble-shooting

- Check that the card is properly installed in the PC.

- Check that the ADSL connection is active. If the system tray icon is red, click the “Start modem” button on the “General” page of the “ADSL Analyzer”.

- If the icon stays red, check the ISP provided modulation parameter against the ADSL mode in the configuration.  Stop the modem and configure it to match the ISP.

- If the icon is green but the internet connection is not active , stop the modem and check that the ISP- connection related parameters like “Encapsulation”, “VPI” and “VCI” are configured correctly. Re-configure those to match the settings provided by the ISP. Also, checks Username, Password are as specified by the ISP.

- If the problem is not solved, click the “Generate log” button on the “General” page of the “ADSL Analyzer”. A log file “modem.dat” is created in the “Install” directory. Mail the file to the support team.

Windows 98/Millenium ONLY:

- After you make an initial Dial-Up Networking connection on an Asynchronous Transfer Mode (ATM) adapter and then end the connection, additional attempts to connect may not work, and you may receive the following error message: “Error 602: The modem is being used by another Dial-Up Networking connection or another program. Disconnect the other connection or close the program, and then try again”. This is a known problem with the operating system. (REF: http://support.microsoft.com/?id=271701).

- The machine may hang on shutdown. This is not a major problem as the system will recover if the power is cycled. This is a known problem with the operating system. (REF: http://www.microsoft.com/windows98/downloads/contents/WURecommended/S_WUFeatured/Win98SE/Default.asp ).

Windows XP – Service Pack 2 ONLY:

- You may receive a "Limited or no connectivity" message on a computer that is connected to the Internet by a DSL modem or a cable modem after you install Windows XP Service Pack 2 (REF:  http://support.microsoft.com/?kbid=892896 ).