Configuring an RTA1046 with an Engin VoIP account.

Before proceeding you will need to complete the setup of all required hardware components as per the User Guide that came supplied with your Dynalink RTA1046.

- Open up web-browser and type in 192.168.1.1 in the address bar and press <Enter>

- Now enter in the modem user name password.  By default the username is “admin” and the password is “admin” (without quotes).

- Click on “Voice” on the top menu bar.

- Now type the following information as required

  - Interface 			ppp_8_35_1 (from drop down menu)

  - SIP Registrar Address		syd.byo.engin.com.au

  - Port	 			5060

  - SIP Outbound Proxy		syd.byo.engin.com.au

  - Port				5060

  - SIP Proxy Server Address	syd.byo.engin.com.au

  - Port				5060

  - Phone Number			Engin Number

  - Caller Name			Engin Number

  - Username 			Engin Number

  - Password			Engin Password

Note: - leave rest of the setting as default settings.

- Now click on “Apply”.

- The RTA1046VW will now attempt to register to Exetel and you will be presented with the following screen.

  - You should now be presented with the below screen confirming that your registration details have been successful

Congratulations, you can now start using a standard telephone handset that is connected to the Dynalink RTA1046VW using Engin’s VoIP service.

Happy chatting!