Configuring an RTA1046 with an Exetel VoIP account.

Before proceeding you will need to complete the setup of all required hardware components as per the User Guide that came supplied with your Dynalink RTA1046.

- Open up a web-browser and type in 192.168.1.1 in the address bar and press <Enter>

- Now enter in the modem user name password.  By default the username is “admin” and the password is “admin” (without quotes).

- Click on “Voice” on the top menu bar.

- Now enter the following information

- Interface 			ppp_8_35_1 (from drop down menu)

- SIP Registrar Address	sip1.exetel.com.au

- Port	 		5060

- SIP Outbound Proxy		sip1.exetel.com.au

- Port			5060

- SIP Proxy Server Address	sip1.exetel.com.au

- Port			5060

- Phone Number		Exetel Number (as advised by Exetel)

- Caller Name		Exetel Number (as advised by Exetel)

- Username 			Exetel Number (as advised by Exetel)

- Password			Exetel Password (as advised by Exetel)

Note: - leave rest of the setting as default settings.

- Now click on “Apply”.

- The RTA1046VW will now attempt to register to Exetel and you will be presented with the following screen.

  - You should now be presented with the below screen confirming that your registration details have been successful

Congratulations, you can now start using a standard telephone handset that is connected to the Dynalink RTA1046VW using Exetel’s VoIP service.

Happy chatting!