How to Port Forward

Please follow the instructions below to create a Port Forwarding rule on the NB9WMAXX:

- Open your Web Browser (such as Internet Explorer), and enter the following numbers into the Address Bar:
192.168.1.1
If asked to login, the default username and password is "admin" and "admin" without the quotes.

- Click on "Advanced" on the left menu.

- Click on "NAT" on the left menu.

- Click on "Port Forwarding" on the left menu.

- Press the "Add" button.

- Under Server Name, select the "Custom Server" option.
Enter a name for your Port Forwarding rule. This can be anything, is only used to describe the port forwarding rule

- In the Server IP Address box, enter the IP Address of the computer or device you with to Port forward to.
By default, the IP Address will be 192.168.1.x, where 'x' is a number between 2 and 254.
If you are unsure of the IP Address of your computer, or the address on your computer is "dynamic", please see the following page for more information about setting a "Static IP":
http://www.portforward.com/networking/staticip.htm

- Under External Port Start and External Port End on the next blank row of the table, enter the port number that you wish to Port Forward.
If you are entering a range of ports (such as 1000-2000) then you would enter 1000 into External Port Start, and 2000 into External Port End.

- Under Protocol on the same row of the table, select the Protocol that is required for your ports. If you do not know this, select "TCP/UDP".

- Under Internal Port Start and Internal Port End on the same row of the table, enter the port number that the forward ports will be redirected to.
Typically, these are the same numbers as in Step 8.
The same rules apply as in Step 8 if you wish to enter a range of ports.

Note: If you wish to enter more Port Forwarding rules for this IP Address, then you can repeat steps 8-10.

- When you are finished, press the "Save/Apply" button which is located under the table.