Configuring an RTA1046 with an iiNet VoIP account.

Before proceeding you will need to complete the setup of all required hardware components as per the User Guide that came supplied with your Dynalink RTA1046.

- Open up web-browser and type in 192.168.1.1 in the address bar and hit enter

- Now enter in the modem user name password.  By default the username is “admin” and the password is “admin” (without quotes).

- Click on “Voice” on the top menu bar.

- Now type the following information as required

  - Interface 			ppp_8_35_1 (from drop down menu)

  - SIP Registrar Address		sip.”state”.iinet.net.au *

  - Port	 			5060

  - SIP Outbound Proxy		sip.”state”.iinet.net.au *

  - Port				5060

  - SIP Proxy Server Address	iinetphone.iint.net.au

  - Port				5060

  - Phone Number			iiNet Number

  - Caller Name			iiNet Number

  - Username 			iiNet Number

  - Password			iiNet Password

Note: - leave rest of the setting as it is.

* - Enter the correct server address below for the state that you are in into the SIP Registrar Address and SIP Outbound Proxy fields

ACT sip.act.iinet.net.au

NSW sip.nsw.iinet.net.au

NT sip.nt.iinet.net.au

QLD sip.qld.iinet.net.au

SA sip.sa.iinet.net.au

TAS sip.tas.iinet.net.au

VIC sip.vic.iinet.net.au

WA sip.wa.iinet.net.au

- Now click on “Apply”.

- The RTA1046VW will now attempt to register to Exetel and you will be presented with the following screen.

- You should now be presented with the below screen confirming that your registration details have been successful

Congratulations, you can now start using a standard telephone handset that is connected to the Dynalink RTA1046VW using iiNet’s VoIP service.

Happy chatting!