Configuring an RTA1046 with a TPG VoIP account.

Before proceeding you will need to complete the setup of all required hardware components as per the User Guide that came supplied with your Dynalink RTA1046.

- Open up web-browser and type in 192.168.1.1 in the address bar and hit enter

- Now enter in the modem user name password.  By default the username is “admin” and the password is “admin” (without quotes).

- Click on “Voice” on the top menu bar.

- Now type the following information as required

  - Interface 			ppp_8_35_1 (from drop down menu)

  - SIP Registrar Address		aphone3.tpg.com.au

  - Port	 			5060

  - SIP Outbound Proxy		aphone3.tpg.com.au

  - Port				5060

  - SIP Proxy Server Address	aphone3.tpg.com.au

  - Port				5060

  - Phone Number			TPG Number (as advised by TPG)

  - Caller Name			TPG Number (as advised by TPG)

  - Username 			TPG Number (as advised by TPG)

  - Password			TPG Password (as advised by TPG)

Note: - leave rest of the setting as default settings.

- Now click on “Apply”.

- The RTA1046VW will now attempt to register to Exetel and you will be presented with the following screen.

  - You should now be presented with the below screen confirming that your registration details have been successful

Congratulations, you can now start using a standard telephone handset that is connected to the Dynalink RTA1046VW using TPG’s VoIP service.

Happy chatting!